1185-01-07 Manado to Ambon

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy - 18 | Init. Hdg - 181 | Init. Alt – 2,000 ft |  | Apt Elev. – 262 ft |  |  |  |
| Ratulangi Manado (WAMM) Indonesia To Pattimura Ambon (WAPP) Indonesia | Tune ADF to PN NDB 290.0 Departure. To Fix 01. After take off continue Rwy Hdg 181°. Commence climb to 2,000 ft MSL. Waypoint at interception of 333° bearing to station…………………………. En-Route. To HA NDB 290.0. Turn left to 153° (Bearing to station = 333°). Commence climb to 7500ft MSL. When signal fades tune ADF to HA NDB 290.0. (Same freq. as PN NDB). When signal received intercept 153° bearing to station. Direct to NDB…. To OH NDB 340.0. Turn left to 113° Start timer. After ten minutes commence descent to 2,000ft MSL. Slow to 120kts. Direct to NDB…………………………. Approach. To Fix 03. Turn left to 075°. (Bearing to station = 255°). Start timer. Descend to 1,000ft MSL. Waypoint after five minutes (120kts)……………………………… To Rwy. Make a left standard rate turn to 255° and follow coastline. Descend to 500ft MSL. Terrain hazard in airport vicinity. When runway is in sight, turn left onto runway heading 223° for visual approach to runway………………………… Land Pattimura Rwy 22. Length – 6,074 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 181 153 113 075 Final Hdg 225 | 6.0nm 311.0nm 66.0nm 4.0nm 5.0nm | 00+03 02+00 00+29 00+02 00+03 |
| Flight No. 1185-01-07 | Arrival Airport Elev. - 32 ft |  |  | Estimated totals for flight>>> |  |  | 392nm | 02+37 |