1185-01-08 Ambon to Kupang

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy - 22 | Init. Hdg - 223 | Init. Alt – 2,000 ft |  | Apt Elev. – 32 ft |  |  |  |
| Pattimura Ambon (WAPP) Indonesia To El Tari Kupang (WRKK) Indonesia | Tune ADF to OH NDB 340.0 Departure. To Fix 01. After take off continue Rwy Hdg 223°. Commence climb to 2,000 ft MSL. Waypoint at 2,000ft MSL……………………………………………………….. To Fix 02. Turn left to 170°. Commence climb to 6,500ft MSL. Waypoint at interception of 017° bearing to station……………………………………………….. En-Route. To BC NDB 315.0. Turn right to 197° (Bearing to station = 017°). When signal fades maintain Hdg and tune ADF to BC NDB 315.0. When signal received intercept 197° bearing to station. Direct to NDB……………………………………. To AA NDB 300.0. Turn right to 245° (bearing to station = 065°) and start climb to 8,500ft. Start timer. After fifteen minutes tune ADF to AA NDB 300.0° and intercept 245° bearing to station. Direct to NDB……………………………………… To Fix 03. Turn left to 226° (Bearing to station = 046°). Start timer. After fifteen minutes tune ADF to OK NDB 385.0 and intercept 226° bearing to station. Waypoint after thirty minutes (140kts)……………………………………………. To OK NDB 385.0. Maintain Hdg. Commence descent to 3,500ft MSL. Slow to 120kts. Direct to NDB………………………………………………………………….. Approach. To Fix 04. Turn left to 090°. (Bearing to station = 270°). Start timer. Descend to 2,000ft MSL. Waypoint after four minutes. ……………………………………… To Rwy. Make a left standard rate turn to intercept 258° bearing to station. Descend to 800ft MSL. When runway is in sight, turn left onto runway heading 252° for visual approach to runway………………………… Land El Tari Rwy 25. Length – 8,220 ft. Width – 148 ft. Surface – Asphalt. |  |  |  |  | 223 170 197 245 226 226 090 Final Hdg 252 | 4.0nm 4.0nm 297.0nm 95.0nm 78.0nm 20.0nm 8.0nm 9.0nm | 00+02 00+02 01+55 00+37 00+30 00+10 00+04 00+05 |
| Flight No. 1185-01-08 | Arrival Airport Elev. - 334 ft |  |  | Estimated totals for flight>>> |  |  | 515nm | 03+25 |