1185-02-04 Pontianak to Jambi

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy - 15 | Init. Hdg - 158deg. | Init. Alt – 2,000ft |  | Apt Elev.- 9ft. |  |  |  |
| Supadio (WIOO) Indonesia. To Jambi (WIPA) Indonesia | Tune ADF to AT NDB, 245.0. Departure. To Fix 01. After take off maintain Rwy Hdg. commence climb to 2,000ft MSL. Waypoint at 2,000ft MSL………………………………………………………………. To Fix 02. Turn right to 174°. Commence climb to 6,500ft MSL. Waypoint at interception of 076° bearing to station……………………………………………….. En-Route. To Fix 03. Turn left to intercept 256° bearing OB from AT NDB and track (Bearing to station = 056°). Start Timer. When signal fades maintain heading and tune ADF to NX NDB, 365.0. When signal received intercept 256° bearing to station. Waypoint after one hour and fifty minutes, or crossing coast……………………… To NX NDB, 365.0. Maintain heading and commence descent to 2,000ft MSL. Slow to 120kts at end of descent. Direct to NDB……………………………………. Approach. To Fix 04. Turn right to 290° magnetic (Bearing to station = 110°). Start timer. Descend to 1,000ft MSL. Waypoint after three minutes…………………………… To runway. Make a right standard rate turn to 075° and intercept 125° bearing to station. Turn right and track 125° bearing. Descend to 600ft MSL. When runway is visual turn right to runway heading 134°……………………………………………. Land Sultan Taha Rwy 13. Length – 6,577ft. Width – 98ft. Surface – Asphalt. |  |  |  |  | 158deg 076deg 256deg 256deg 290deg 075/125deg | 5.0nm 14.0nm 293.0nm 52.0nm 6.0nm 7.0nm | 00+02 00+06 01+50 00+21 00+03 00+05 |
| Flight No. 1185-02-04 | Arrival Airport Elev. – 85ft. |  |  | Estimated totals for flight>>> |  |  | 377 nm | 02+27 |