1185-02-08 Padang to Palembang

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy - 5 | Init. Hdg - 047deg. | Init. Alt – 1,500ft |  | Apt Elev.- 88ft. |  |  |  |
| Padang (WIMG) Indonesia To Palembang (WIPP) Indonesia | Tune ADF to ON NDB, 375.0. Departure. To Fix 01. After take off maintain Rwy Hdg. commence climb to 1,500ft MSL. Waypoint at 1,500ft MSL………………………………………………………………. To Fix 03. Make a standard rate left turn to 245°. Commence climb to 10,500ft MSL. Start Timer. Waypoint at interception of 257° Bearing Outbound (Bearing to station = 077°) from ON NDB…………………………………………………………. En-Route. To Fix 04. Turn right to intercept 257° bearing OB from AD NDB and track (Bearing to station = 077°). Waypoint when crossing coast. After approximately forty minutes………………………………………………………………………………….. To Fix 05. Turn left to 144°. Start timer. Descend to 9,500ft MSL. Tune ADF to OQ NDB, 295.0. Course roughly follows the coast. Waypoint when OQ NDB is received. After approximately one hour and forty two minutes and seventy five miles from destination…………………………………………………………………… To OQ NDB, 295.0. Intercept 144° bearing to station. Commence 300ft/min descent to 3,500ft MSL. Slow to 120 knots at end of descent. Direct to NDB……. Approach. To Fix 07. Make a right standard rate turn to 340°. Start timer. Descent to 1,200ft MSL. Waypoint after five minutes……………………………………………………... To runway. Make a right standard rate turn to 125° and intercept 168° bearing to station. Turn right and track 168° bearing. Descend to 500ft MSL. When runway is visual turn left to runway heading 161°. Land Padang Rwy 16. Length – 7,088ft. Width – 148ft. Surface – Concrete. |  |  |  |  | 047deg 245deg 257deg 144deg 144deg 340deg 125/168deg | 3.0nm 8.0nm 92.0nm 235.0nm 75.0nm 10.0nm 9.0nm | 00+02 00+03 00+35 01+42 00+35 00+05 00+05 |
| Flight No. 1185-02-08 | Arrival Airport Elev. – 9ft. |  |  | Estimated totals for flight>>> |  |  | 432 nm | 03+07 |