1185-02-08 Padang to Palembang

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy - 16 | Init. Hdg - 161deg. | Init. Alt – 2.000ft |  | Apt Elev.- 9ft. |  |  |  |
| Padang (WIMG) Indonesia To Palembang (WIPP) Indonesia | Tune ADF to OQ NDB, 295.0. Departure. To Fix 01. After take off maintain Rwy Hdg. commence climb to 2,000ft MSL. Waypoint at interception of 140° bearing OB (Bearing to station = 320°)…………. En-Route. To KC NDB, 201.0. Turn left to intercept 140° bearing OB from KC NDB and track (Bearing to station = 320°). Commence climb to 9,500ft MSL. When established on course tune ADF to KC NDB, 201.0. When signal received intercept 140° IB bearing. Direct to NDB….......................................................................................... To Fix 02. Turn left to intercept 104° bearing OB from KC NDB. (Bearing to station = 284°. When signal fades maintain heading and tune ADF to OW NDB, 395.0. Waypoint when signal received. (Approximately seventy five miles from destination)……………………………………………………………………………….. To OW NDB, 396.0. Maintain heading. Commence 300ft/min descent to 3,500ft MSL. At end of descent slow to 120kts. Direct to NDB……………………………… Approach. To Fix 04. Make a left standard rate turn to 293°. Start timer. Commence descent to 1,200ft MSL. Tune ADF to WW NDB, 380.0 and intercept 293° bearing OB (Bearing to station = 113°) Waypoint after three minutes………………………….. To runway. Make a right procedure turn. Turn right to 338° and fly heading for one minute. Make a left standard rate turn to 158°. Intercept and turn to 113° bearing IB. Descend to 600ft MSL. When runway in sight, make normal visual approach and landing………………………………………………………………………………. Land Sultan M Badaruddin Li Rwy 11. Length – 7,227ft. Width – 148ft. Surface – Asphalt. |  |  |  |  | 161deg 140deg 104deg 104deg 293deg 158/113deg | 5.0nm 91.0nm 129.0nm 75.0nm 6.0nm 14.0nm | 00+03 00+40 00+56 00+35 00+03 00+08 |
| Flight No. 1185-02-08 | Arrival Airport Elev. – 36ft. |  |  | Estimated totals for flight>>> |  |  | 320 nm | 02+25 |