| FROM - TO | Flight Description “Allocated runways and related information may change when flying online or using Real Weather” |  |  |  |  | COURSE (Leg) | DISTANCE (Leg) | ETE (Leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy – 26 | Init. Heading – 257 deg. |  | Init. Alt. – 7500 ft | Apt Elev. - 775 ft |  |  |  |
| Burbank (KBUR) California, USA To Las Vegas (KLAS) Nevada, USA | Departure: Fly Rwy heading NDB UR (253.0)…………………………………….. …….. On station passage, track direct to VOR VNY (113.10)……………………………… Enroute: On VNY station passage turn right to course 278 tracking VOR FIM (112.50) radial 098 inbound.……………………………………………………………………………. At FIM station passage, turn right to course 053 and track VOR PMD (114.50) radial 233 inbound. ……………………………………………......................................... At PMD station passage turn right to course 068 and track the PMD 068 radial outbound. At PMD DME 40.0, tune VOR HEC (112.70) and track 249 radial inbound……….. At BASAL, turn left to course 034. Commence Track inbound on VOR DAG (113.2) radial 214…………………………………………………………………………………….. At DAG station passage, turn left to course 031 and track DAG radial 031 outbound. Commence climb to 9,500ft MSL……………………………………………… Approach: At RDSP reduce speed to 130 knots, begin 500 FPM descent to 4,500ft MSL. Turn right to course 032, track VOR LAS (116.9) radial 212 inbound..........….. At OASYS, turn right course 064, reduce speed to 120 knots. Maintain 500 FPM descent. Track VOR BLD (116.70) radial 244 inbound………………………………….. At DCAR turn left to course 011. Continue descent at pilot discretion to Rwy 1L… Land Las Vegas Rwy 1L. Length – 9,877ft. Width – 150ft. Surface – Concrete. |  |  |  |  | 257deg 271deg 278deg 053deg 068deg 034deg 031deg 032deg 064deg 011deg | 1.4nm 5.9nm 20.9nm 43.6nm 59.8nm 18.6nm 62.7nm 18.5nm 6.5nm 9.8nm | 00+01 00+03 00+09 00+17 00+23 00+07 00+24 00+07 00+03 00+06 |
| Flight No. 729-01-03 | Arrival Airport Elevation: 2178ft |  | Estimated Totals for Flight>>> |  |  |  | 248nm | 01+39 |

NOTES:  Departure is based on IFR Departure Procedure Rwy 26, this is NOT an IAP for KLAS. If flying real weather, use only if ceiling 8,000 MSL or greater and visibility 3 nm. Enroute altitudes are VFR, based on MEA’s if  appropriate airways are available. BASAL is defined by intersection of  VOR HEC (112.7) radial 068 and VOR DAG (115.2) radial 214, RDSP VOR LAS (116.9) radial 212 @ 30.0 DME,  OASYS by VOR LAS (116.9) radial 212 and VOR BLD (116.70) radial 244, DCAR by VOR LAS (116.9) radial 193 and VOR BLD (116.70) radial 244.