| FROM - TO | Flight Description “Allocated runways and related information may change when flying online or using Real Weather” |  |  |  |  | COURSE (Leg) | DISTANCE (Leg) | ETE (Leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy – 26 | Init. Heading – 257 deg. |  | Init. Alt. – 7500 ft | Apt Elev. - 775 ft |  |  |  |
| Burbank (KBUR) California, USA To Reno (KRNO) Nevada, USA | Departure: Fly Rwy heading NDB UR (253.0)…………………………………….. On station passage, turn right direct to VOR VNY (113.10)………………………… Enroute: On VNY station passage turn right to course 278 tracking VOR FIM (112.50) radial 98 inbound.……………………………………………………………………………. At FIM passage, turn right to course 053, track VOR PMD (114.50) radial 233 inbound………………………………………………………………………………………… At PMD station passage turn right to course 068 and track the PMD 068 radial outbound. At PMD DME 40.0, tune VOR HEC (112.70) and track 248 radial inbound………………………………………………………………………………………… At BASAL, turn left to course 034.Track inbound on VOR DAG (113.2) radial 214. At DAG station passage, turn left to course 341 and track DAG radial 341 outbound. Commence climb to 8,500. At DAG DME 50.0 tune VOR BTY (114.70) and track inbound on radial 161…………………………………………………………..……… At BTY passage turn left to course 335 and track BTY radial 335 outbound. At BTY DME 30.0 tune VOR TPH (117.2) and track inbound on radial 155……………. At TPH station passage turn left to course 289, track inbound on 109 radial VOR MVA (115.1). Climb to 10,500ft MSL……………………………………………………… At MVA station passage, turn right to course 292, track 292 radial MVA outbound. At MVA DME 50.0 tune VOR FMG (117.9) and track 112 radial inbound…………… Approach: AT FMG DME 8.0 commence 500 FPM descent to 9000ft MSL………… At FMG station passage, turn right to course 316 tracking outbound on FMG 316 radial. Tune KRNO ILS runway 16R (110.9, 164 degrees)…………………………. At TAKLE, turn right to 344, establish on ILS backcourse, descend at 500 FPM to 8,500ft MSL. Once established 1 min on backcourse, turn left to 299 degrees. Fly 299 degrees for 1 minute, then turn right 180 degrees to a course of 119. When CDI comes off peg, turn right to 164 and establish on localizer………………………………. Land Reno Rwy 16R. Length – 11,008ft. Width – 150ft. Surface – Concrete. |  |  |  |  | 257deg 271deg 278deg 053deg 068deg 034deg 341deg 335deg 289deg 292deg 292deg 316deg 344/164deg | 1.4nm 5.9nm 20.9nm 43.7nm 59.8nm 18.6nm 110.6nm 75.1nm 56.9nm 87.7nm 7.6nm 11.2nm 19.5nm | 00+01 00+03 00+09 00+17 00+23 00+07 00+42 00+28 00+21 00+32 00+04 00+02 00+08 |
| Flight No. 729-01-05 | Arrival Airport Elevation: 4409ft. |  | Estimated Totals for Flight>>> |  |  |  | 519nm | 03+21 |

NOTES:  Departure is based on IFR Departure Procedure Rwy 26, Approach based on ILS Rwy 16R for KRNO. MAP is climb to 6,500 then climbing left to 11,000 direct to FMG, hold. Reminder, CDI on ILS backcourse is reversed, if needle left of center turn RIGHT to center. TAKLE intersection of FMG radial 316 and KRNO ILS localizer.