| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy -28 | Init. Hdg – 036deg | Init. Alt – 3,000ft |  | Apt Elev. – 531ft |  |  |  |
| Presque Isle, Maine, USA. (KPQI) to Goose Bay, Newfoundland, Canada. (CYYR) | Departure. To YSL NDB, 404.0. After take off make a right turn to 036deg and commence climb to 3,000ft MSL……………………………………………………………………. Enroute. To YVB NDB, 243.0. On station passage YSL turn right to 081deg and commence climb to 7,000ft MSL…………………………………………………….. To GP NDB, 232.0……………………………………………………………………. To NA NDB, 385.0……………………………………………………………………. Track to YR NDB, 257.0. When signal is lost from NA maintain Hdg 043deg until YR NDB is received, then continue Hdg for a further five minutes………………. Approach. To YR NDB, 257.0. Commence a 500fpm descent to 3,000ft MSL……………… To runway. On station passage YR turn right to Hdg 053deg for two minutes and commence a 400fpm descent to 500ft MSL. Make a standard rate 210deg right turn to runway Hdg 263deg. Visual approach to runway…………………………… Land Goose Bay Rwy 26. Length – 10,841ft. Width – 157ft. Surface – Concrete. Missed approach. Climb straight ahead to 3,000ft MSL then turn left until heading direct to YR NDB. Repeat approach. |  |  |  |  | 036deg 081deg 073deg 072deg 043deg 043deg 263deg | 35.2nm 107.5nm 61.6nm 132.1nm 174.4nm 20.4nm 9.6nm | 00+17 00+42 00+24 00+51 01+07 00+09 00+06 |
| Flight No.196-01-01 | Arrival Airport Elev. – 157ft |  |  | Estimated totals for this flight>>> |  |  | 541nm | 03+34 |