| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy -26 | Init. Hdg – 101deg | Init. Alt – 1,500ft |  | Apt Elev. – 157ft |  |  |  |
| Goose Bay, Newfoundland Canada. (CYYR) To Narsarsuaq, Greenland. (BGBW) | Departure. To YR NDB, 257.0. After take off make a right turn to 101deg and climb to 1,500ft MSL. Fly direct to YR NDB…………………………………………………………………… Enroute. To JC NDB, 396.0. Climb to 5,000ft MSL………………………………………………… Track to SI NDB, 279.0. Climb to 7,000ft MSL. Read the accompanying notes for advice on how to fly this leg. When signal from JC NDB fades tune ADF to SI NDB. When signal received make any necessary correction to your course and continue for a further five minutes……………………………………………………………………………. Approach. To SI NDB, 279.0. Commence a 500fpm descent to 3,000ft MSL…………………….. To NS NDB, 404.0. Commence a 400fpm descent to 1,000ft MSL…………………….. To runway. After station passage NS initially maintain heading and descend to 500ft MSL. Refer to the accompanying notes for detailed guidance on the approach to this difficult airport...................................................................................................... Land Narsarsuaq Rwy 7. Length – 6039ft. Width – 148ft. Surface – Concrete. Missed approach. For detailed procedure refer to the accompanying notes. |  |  |  |  | 110deg 082deg 073deg 073deg 085deg (Mean Course 083deg) | 9.3nm 85.0nm 524.4nm 20.5nm 21.4nm 24.3nm | 00+05 00+35 03+22 00+08 00+10 00+14 |
| Flight No.196-01-02 | Arrival Airport Elev. – 111ft |  |  | Estimated totals for this flight>>> |  |  | 685nm | 04+34 |