# Leg 2 Goose Bay to Narsarsuaq 196-01-02

## Please set the Weather in two stages:

LOCAL WEATHER  (for the conditions at Goose)

Clouds				Overcast

Precipitation			None

Visibility			5 miles

Wind Speed (strength)	Light

Wind Direction		250 degrees approx. (Anywhere roughly southwest)

### Advanced Weather

Cloud type		Stratus        2500  –  4000 ft.      Deviation  200 ft.

Coverage		Overcast  8/8

Turbulence		None

Precipitation type	None

Rate:			None

GLOBAL WEATHER  (for conditions later in the journey)

Clouds			Overcast

Precipitation		Light

Visibility		3 miles

Wind Speed		Light

Wind Direction	235 degrees approx.

#### Advanced Weather - CLOUDS

Cloud type		Stratus       1000 ft  –  4000 ft       Deviation  200 ft.

Cloud Coverage	Overcast  8/8

Precipitation type	Rain

Base altitude		-1499   (Default setting  – makes sure it rains down to sea level)

Rate			Light (low)

Turbulence		Occasional

Icing			None

###### Leave WIND, TEMP/PRESS and VISIBILITY tabs as they are.

Once you are clear of Goose Bay, the conditions should deteriorate from the Local settings at Goose, to the Global settings. If you prefer, you can just use the Global settings, but vary them to the following, when you have passed Rigolet:

Visibility to 3 miles

Clouds to: 1000 – 4000 ft

Precipitation: Light rain

Occasional turbulence

#### GOOSE  (Goose Bay) CYYR

##### Depart  Runway 26

500 ft above airfield turn right for Goose NDB (Ident. YR; freq. 257)

Cleared initially to 1500 ft.

#### GOOSE NDB  257

Set course for Rigolet NDB (Ident. JC; Freq. 396)

After Goose NDB cleared to 5000 ft.

Required track: 082 deg.

Maintain track by watching back bearing to Goose, and balancing any drift with a corresponding adjustment to heading, until Rigolet NDB is acquired.

#### RIGOLET NDB  396

##### Set course for Simiutaq NDB (Ident. SI; Freq. 279)

Required track: 073 deg.

After Rigolet NDB   cleared to final cruising height of 7000 ft.

When Simiutaq NDB has been acquired, continue on track for five minutes, then descend to 3000 ft.

****  Please read section four of the accompanying “Long Range Navigation Notes” ****

This is a long East-West leg, so you need to be aware that the required track is the INITIAL track only, for a Great Circle route, and that to follow a Great Circle route you need to change heading throughout the leg.

In this case the Conversion Angle is 5 degrees, so you can either, over the duration of the flight, steadily alter heading from the initial 73 deg. track to the final track of 83 deg.

(Twice the Conversion angle), or instead fly a Rhumb Line track, which is a constant direction.

The Rhumb Line track is the initial Great Circle track plus the Conversion angle, i.e. 78 deg.

If you decide to follow the Great Circle route, as the expected duration of the leg is about four hours; after one, two and three hours flight, you could alter your heading by three, four and three degrees to the right, respectively.

To find out how to work out “Conversion Angle” please see the notes.

It is only relevant for long flights crossing a number of meridians, and is worked out from the Change of Longitude, and the Mean (or Average) Latitude.

An “abac” is provided to make this easy.

#### SIMIUTAQ NDB  279

Set course for Narsaq NDB  (Ident.NS; Freq. 404)

Required Track: 085 deg.

After you have passed Simiutaq NDB, descend to 1000 ft.

#### NARSAQ NDB  404

At Narsaq NDB maintain heading and descend to 500 ft., or lower if necessary to break clear of cloud, down to the Company Minimum; to fly a visual approach to Bluie West One.

Visual Approach to Bluie West One, in poor visibility.

Narsaq NDB  lies on the North shore of the entrance to the fjord.

It is essential in bad visibility to keep the North, or Left shore in sight at all times.

Follow the coastline, keeping just to the seaward side of the shore, in a slow cruising configuration.

5 miles into the fjord, ignore the inlet to the left. Bear right to pick up the main coastline, which continues in its original Easterly direction.

Tune the Radio Compass to Narsarsuaq NDB (Ident. NA; Freq. 359),

but DO NOT head directly for it at this point.

After a further 6 miles, the coastline bends gently to the left, and after another 8 miles turns more sharply until it lies on a heading of 020 deg. Set landing configuration.

At this point, continue flying the heading 020 deg., and watch the needle of the Radio Compass.  When it indicates a bearing of 050 deg to the right, turn Right onto a heading of 070 deg. The Radio Compass should now indicate straight ahead.

Be prepared to anticipate this, so that as you roll out onto the heading, the runway should appear dead ahead.

Note that the NDB is not on the runway centreline, but lies to the left of it.

In real life, as the runway slopes upwards to the East – towards the rising ground of the glacier, landings are normally made to the East, and take-offs to the West.

(Note that in FS2002 there is a low escarpment at the edge of the airfield, so if you are making a really low approach, under the cloud base, you may have to climb to see the runway.)

Missed Approach.

This is not a good place to miss your approach in poor visibility, surrounded as you are by high ground, and if the weather is so bad that a safe approach is impossible, the preferred option would be to return to Goose.

However, if strong headwinds make this impossible, as a final resort you could attempt a let down to sea level over the main area of the fjord using the Narsarsuak NDB.

If the visual approach has to be aborted, make a maximum rate-of-climb to 3500 ft and head for the Narsarsuak NDB.

Overhead the NDB continue on an Easterly heading for two minutes, and make a rate 1 turn to the left, to head back to the NDB.

Overhead the NDB, turn onto a heading of 220 deg., and start a steep but slow descent, at 90 kts., with full flap and undercarriage down, to 500 ft, or to within sight of the sea. Ease the rate of descent as you approach sea level. (You don’t have to worry about passengers’ ears popping, as this is a freight flight - on the return journey you will be ferrying aircrew home, but they will appreciate the problem, and you getting them down in one piece.)

Turn right onto a heading of 020 deg., and watch the Radio Compass needle until it shows a bearing of 050 deg. Right.

Turn Right onto a heading of 070 deg., which should centre the needle, and fly towards the NDB. This should take you over the end of the runway.