| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy -25 | Init. Hdg – 200deg | Init. Alt – 4,000ft |  | Apt Elev. – 111ft |  |  |  |
| Narsarsuaq (BGBW) Greenland. To Reykjavic (BIRK) Iceland. | Departure. To NS NDB, 404.0. After take off turn left to 200deg and commence climb to 4,000ft MSL. Tune ADF to NS NDB. When indicated bearing to station is 270deg (70deg right) make a right turn to 240deg. When indicated bearing to station is 280deg (40deg right) turn right to that heading and fly direct to NDB…………………………………………….. Enroute. To JH NDB, 265.0……………………………………………………………………………… To NN NDB, 270.0. Climb to 7,000ft MSL…………………………………………………… To OZN NDB, 372.0. Climb to 9,000ft MSL…………………………………………………. Track to KF NDB, 392.0. Refer to accompanying notes for advice on flying this leg. When signal from KF NDB is received correct your heading and continue for a further five minutes……………………………………………………………………………. Approach. To KF NDB, 392.0. Commence a 500fpm descent to 3,000ft MSL……………………… To Fix 05. On station passage KF turn left to 076deg. Commence a 500fpm descent to 600ft MSL. When established on course tune ADF to NS NDB, 370.0………………….. To runway. When indicated bearing to station is 138deg make a right turn to runway heading 138deg. Visual approach to runway. Beware the NDB mast close to Rwy threshold………………………………………………………………………………………… Land Reykjavic Rwy 14. Length – 4,754ft. Width – 148ft. Surface – Asphalt. Missed approach. Maintain runway Hdg and climb to 3,000ft MSL. Tune ADF to KF NDB, 392.0. Turn right and fly to KF NDB. O/Head KF turn right to 076deg and recommence approach. |  |  |  |  | Final Course 280deg 215deg 177deg 124deg Init GC Hdg 089deg 099deg 076deg 138deg | 26.7nm 10.5nm 41.7nm 62.8nm 618.1nm 32.9nm 19.3nm 5.1nm | 00+12 00+04 00+16 00+24 03+39 00+13 00+10 00+03 |
| Flight No.196-01-03 | Arrival Airport Elev. – 42ft |  |  | Estimated totals for this flight>>> |  |  | 784nm | 05+01 |