# Leg 3 Narsarsuaq to Reykjavic 196-01-03	      Time of Day: Dawn      Season: Winter

Please set the weather in two stages:

LOCAL WEATHER – 2 areas:

(for the conditions at Bluie-West-One and Julianehaab.)

With the aircraft situated at Narsarsuaq (Bluie West One), select:

### World / Weather / Local Weather

And ensure that under the map it states: “Nearest Reporting Station – Narsarsuaq”

The code BGBW for the airfield on the map should be red.

Set the following weather:

## Clouds			Scattered

Precipitation		None

Visibility		10 miles

#### Wind Speed 		Moderate

Wind Direction	210 deg approx.

## Advanced Weather

Cloud type		Cumulus 	4000 – 7000 ft.	Deviation  300 ft.

Coverage		Scattered 3/8

Turbulence		Occasional

Precipitation type 	None

#### Rate			None

#### Click “OK”

Then set the Local Weather for Julianehaab

Julianehaab is situated just to the South of the inlet to the fjord that leads to Narsarsuaq – Two clicks to the left and one down, on the weather map, should put it in view.)

Click on the small triangle by the code letters BGJH, and the code will turn red, and the text under the map should read: “Nearest reporting Station: Julianehaab, Greenland”.

Repeat the weather, by entering the same conditions as above, in the Weather, and Advanced Weather boxes.

Click “OK”, then set:

GLOBAL WEATHER   (for conditions later in the journey)

Clouds			Overcast

Precipitation		Moderate

Visibility		2 miles

Wind Speed		Moderate

Wind Direction	210 deg. Approx.

### Advanced Weather – CLOUDS

Cloud type		Stratus		500 ft – 4000 ft 	Deviation 200 ft.

Cloud Coverage	Overcast  8/8

Precipitation type	Rain		Rate:	Moderate

Base altitude		-1499 (default setting as before)

Rate			Moderate

Turbulence		Occasional

Icing			None

Add second Cloud Layer:

Cloud Type		Cumulus	11000 – 14000 ft	Deviation 200 ft

Cloud Coverage	Scattered 3/8

Precipitation Type		None

Leave WIND, TEMP/PRESS and VISIBILITY tabs as they are.

Click “OK”, “OK”, to return to the aircraft.

___

Bluie West One (Narsarsuaq) BGBW

Tune ADF to Narsaq NDB 404, but DO NOT  head directly for it at this stage.

## Depart Runway 25

## Cleared initially to climb to 4000 ft.

When above 500 ft. and over the water, turn Left to 200 deg.

When indicated bearing to station reads 270deg (70deg right) turn Right heading 240deg.

When indicated bearing to station reads 280deg (40deg right) turn right heading 280deg.

This will put you on the 280deg. QDM.

### NARSAQ NDB 404

## Turn Left on course for Julianehaab NDB 265

Required Track 215 deg.

### JULIANEHAB NDB 265

## After Julianehab NDB Cleared to 7000 ft.

Turn Left for Nanortalik NDB 270.

Required Track: 177 deg.

(Expect maximum cross-wind on this leg.  Maintain track by watching back bearing to Julianehab NDB, and balancing drift with a corresponding adjustment to Heading.)

### NANORTALIK NDB 270

After Nanortalik NDB cleared to final cruising height of 9000 ft.

Turn Left for Prins Christian Sund NDB 372

Required Track: 124 deg.

PRINS CHRISTIAN SUND NDB 372

Turn Left for Keflavik NDB Iceland. 392

Required Initial GC Track is 089deg. Final GC track is 099deg.

This is the longest leg of the flight, when you will be out of range of radio beacons for several hours.  It is vital that you use the time that you are still in contact with Prins Christian Sund NDB to work out the drift accurately in order to make a good landfall in Iceland.

A very Important Note. It has become very apparent during the development of this charter that FSNavigator is subject to gross errors with regard to its depiction of headings related to long distance flying in the northern latitudes, and no doubt the southern latitudes too.

This takes the form of displaying wrong headings on the FSNav map for Great Circle routes. It should show the initial GC course you must follow but instead displays a totally irrelevant figure. This appears to be a bug in FSNavigator.

It is important that you ignore the figure displayed and follow the directions in the notes for this flight and for those following. After, of course, reading the ADF Tutorial.

The following figures were computed with the aid of the E6-B computer and have been checked and tested.

Initial GC Hdg = 089deg.

Final GC Hdg = 099deg.

Rhumb Line Hdg = 094deg.

A much easier alternative is to follow the slightly longer Rhumb Line route which requires that you follow a single composite heading of 094deg. This heading is guaranteed to deliver you to within capture range of KF NDB. Responsibility for the compensation of winds and weather en route is entirely yours of course.

When Keflavik NDB 392 has been acquired, continue on course for five minutes, then descend to 3000 ft.

### KEFLAVIK NDB 392

After passing Keflavik NDB, set course for Fix 05.

Required Track: 076 deg.

(You will pass close to Keflavik airfield, and may receive inner and outer marker signals soon after passing the Keflavik NDB,)

Commence a 500fpm descent to 600ft. MSL.

Tune ADF to NS NDB, 370

### REYKJAVIK NDB 355

When indicated bearing to station is 138deg turn Right, Heading 138deg. for the NES (Reykjavik) NDB  370,

which is at the end of Reykjavik Runway 14.

Descend clear of cloud. This very low approach will be below the normal glide-path, and you may well see four red lights on the VASIs.

The only obstruction is the NDB aerial itself, which is very close to the runway threshold, and in line with it.

### REYKJAVIK Iceland BIRK

Land Runway 14

(This will be a downwind landing, but this is compensated for by the easier approach in low visibility.)

Note: If these conditions are thought bad, they do not compare with those encountered by E.K. Gann on his first trip to Iceland. There had been no weather reports for 24 hours, and they encountered a lowering cloud base, with no possibility of returning to Bluie-West-One. As they descended through the thick cloud towards the sea, the radio operator trailed an antenna wire with a small weight, or “fish” on the end, and kept a hand on the end of the wire to feel when it struck the water, to give some slight warning. They broke cloud as the weight hit the water – and it only hung down fifty feet…

On top of that, the control tower would not answer their calls, as they had been given the wrong code for the day, and in fact fighter aircraft had been sent up to shoot them down, as they had been reported as “approaching enemy aircraft”.

Coming in under cloud at fifty feet above the sea, they hadn’t been spotted.

Their important cargo – mainly toilet paper, with some bundles of brooms, two filing cabinets and forty metal waste-baskets, plus a misdirected aircraft engine.