| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy -32 | Init. Hdg – 157deg | Init. Alt – 3,000ft |  | Apt Elev. – 42ft |  |  |  |
| Reykjavik (BIRK) Iceland. To Prestwick (EGPK) United Kingdom | Departure. To EL NDB, 335.0. After take off turn right to 157deg. Climb to 3,000ft MSL. Fly direct to EL NDB……………………………………………………………………………………… Enroute. To VM NDB, 375.0……………………………………………………………………………. To BRR NDB, 316.0. After station passage VM turn left to the 149deg bearing OB from VM NDB. Refer to the accompanying notes for help on how to fly this leg. Climb to 9,000ft MSL…………………………………………………………………………………….. To RN NDB, 293.0. Descend to 5,000ft MSL………………………………………………. To CBL NDB, 380.0…………………………………………………………………………… Approach. To PIK NDB, 355.0. Five minutes after station passage CBL NDB commence a 400fpm descent to 2.000ft MSL. Slow to 120kts……………………………………………………. To runway. Overhead Prestwick PIK NDB turn right heading 120 deg. Set landing configuration Start descent at 300 fpm until clear of cloud, but do not descend below 500 ft., maintaining the heading of 120 deg for two minutes. Turn Right heading 220 deg until ADF shows a reading of 100 deg. Right. Turn Right onto a heading of 320 deg. The NDB should now be dead ahead. Runway 31 is to the left of the beacon………………………………………………………………………………………….. Land Prestwick Rwy 31. Length – 9,779ft. Width – 151ft. Surface – Asphalt. Missed approach. Climb to 2000 ft. Fly on a westerly heading for four minutes, and turn right to head back to Prestwick PIK NDB. Repeat approach, but maintain heading of 120 for four minutes, to lengthen each leg, and give extra time to get settled on the heading. Failing this, approach from the sea, via Prestwick PW NDB 426. on a heading of 135 deg. for Runway 13. This will be downwind, but there are fewer obstructions over the sea to worry about. |  |  |  |  | 157deg 155deg Init GC Hdg 149deg 168deg 125deg 092deg Final Hdg 300deg | 9.1nm 56.8nm 539.2nm 65.9nm 31.4nm 38.0nm 9.5nm | 00+04 00+23 03+23 00+34 00+12 00+15 00+05 |
| Flight No.196-01-04 | Arrival Airport Elev. – 62ft |  |  | Estimated totals for this flight>>> |  |  | 771nm | 04+58 |

An Important note for FS2000 users.  CBL NDB is not available. It is suggested that time of passing RN NDB be noted, then at a suitable time initiate a 400fpm descent to 2,000ft MSL. Tune to PIK NDB and correct heading to suit when signal received.