# Leg 4 Reykjavik to Prestwick 196-01-04	         Time of Day: Dawn	     Season: Winter

Please set the weather in two stages:

LOCAL WEATHER – 2 areas:

(for the conditions at Reykjavik and Vestmannaeyjar.)

With the aircraft situated at Reykjavik, select:

### World / Weather / Local Weather

And ensure that under the map it states: “Nearest Reporting Station – Reykjavik, Iceland”

The code BIRK for the airfield on the map should be red.

Set the following weather:

## Clouds			Few

Precipitation		None

Visibility		10 miles

#### Wind Speed 		Moderate

Wind Direction	215 deg approx.

## Advanced Weather

Cloud type		Cumulus 	2500 – 4500 ft.	Deviation  300 ft.

Coverage		Few (2/8)

Turbulence		None			Icing		None

Precipitation type 	None

#### Rate			None

#### Click “OK”

Then set the Local Weather for Vestmannaeyjar.

Vestmannaeyjar is an island off the South coast of Iceland.  Eight clicks to the right, and two down, from Reykjavik, should put it in the centre of the map.

Click on the small triangle by the code letters BIVM, and the code will turn red, and the text under the map should read: “Nearest reporting Station: Vestmannaeyjar, Iceland”.

Repeat the weather, by entering the same conditions as above, in the Weather, and Advanced Weather boxes.

Click “OK”, then set:

GLOBAL WEATHER   (for conditions later in the journey)

Clouds			Overcast

Precipitation		Heavy

Visibility		2 miles

Wind Speed		Moderate

Wind Direction	215 deg.

## Advanced Weather

Cloud type		Stratus 	1500 – 3500 ft.	Deviation  300 ft.

Coverage		Overcast (8/8)

Turbulence		Moderate

Precipitation type 	Rain

#### Rate			Heavy

Wind and Temp/Press tabs remain on default settings, but click on the

Visibility tab, and limit the effective zone of poor visibility to:

Tops		3500 ft

Base		0 ft.

by dragging the blue box, or entering the figures in the windows.

Click “OK”, “OK”, to return to the cockpit.

REYKJAVIK  BIRK

Tune ADF to Ellidavatn NDB 335.

## Depart Runway 32

## Cleared initially to climb to 3000 ft.

When 500 ft. above airfield turn Right for Ellidavatn  NDB  335.

### ELLIDAVATN NDB 335

Set course for Vestmaanaeyjar NDB 375

Establish and maintain required track of 155 deg. by watching the back bearing of Ellidavatn NDB until Vestmaanaeyjar NDB is acquired.

### VESTMAANAEYJAR NDB 375

## After Vestmaanaeyjar NDB cleared to 9000 ft.

Set course for Barra NDB  316

As this is a very long leg, most of the time outside the range of radio navigation aids, maintain track carefully while Vestmaanaeyjar is still in range, by watching the back bearing, and balancing drift with a corresponding alteration of heading.

Initial Great Circle Hdg = 149deg.

Final Great Circle Hdg = 150deg.

Rhumb Line Hdg = 150deg.

The Conversion Angle is 5 deg., so the corrected Rhumb Line track is 150 deg., and the corrected Great Circle track changes from 149 deg to 150 deg over about three and a half hours.

The heading’s stated are calculated using the E6-B computer and have been thoroughly checked and tested.

##### BARRA NDB  316

(If Barra NDB does not appear after the expected flight time, and you suspect that you may have been blown considerably off course, check also the frequencies of Stornoway NDB 431 and Tory Island Lt.Ho. NDB 313.)

After Barra NDB descend to 5000 ft. and set course for Rhinns of Islay RN NDB 293

Required track: 168 deg.

RHINNS OF ISLAY RN NDB 293

Set course for Campbeltown NDB 380

Required track: 125 deg.

##### CAMPBELTOWN NDB 380

Set course for Prestwick NDB 355

Required track 091 deg.

5 minutes after Campbeltown NDB descend to 2000 ft.

##### PRESTWICK PIK NDB 355

Overhead Prestwick PIK NDB turn right heading 120 deg.

Set landing configuration

Start descent at 300 fpm until clear of cloud, but do not descend below 500 ft.,

maintaining the heading of 120 deg for two minutes.

Turn Right heading 220 deg until ADF shows a reading of 100 deg. Right.

Turn Right onto a heading of 320 deg.

The NDB should now be dead ahead.

PRESTWICK EGPK Runway 31 is to the left of the beacon.

For a missed approach, climb to 2000 ft.

Fly on a westerly heading for four minutes, and turn right to head back to Prestwick PIK NDB.  Repeat approach, but maintain heading of 120 for four minutes, to lengthen each leg, and give extra time to get settled on the heading.

Failing this, approach from the sea, via Prestwick PW NDB 426

on a heading of 135 deg. for Runway 13.

This will be downwind, but there will be less obstructions over the sea to worry about.