| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy -31 | Init. Hdg – 313deg | Init. Alt – 4,000ft |  | Apt Elev. – 62ft |  |  |  |
| Prestwick (EGPK) United Kingdom To Blackbushe (EGLK) United Kingdom | Departure. To PW NDB, 426.0. After take off fly direct to NDB. Commence climb to 4,000ft MSL… Enroute. To WFR NDB, 339.0…………………………………………………………………………… To RWY NDB, 359.0. Climb to 7,500ft MSL………………………………………………… To PS NDB, 304.0……………………………………………………………………………… To WPL NDB, 323.0…………………………………………………………………………… To SH NDB, 426.0……………………………………………………………………………… To GST NDB, 331.0. Commence a 500fpm descent to 5,500ft MSL……………………. To BZ NDB, 386.0. Commence a 500fpm descent to 3,500ft MSL………………………. To WOD NDB, 352.0. Five minutes after station passage BZ NDB commence a 400fpm descent to 2,000ft MSL……………………………………………………………… Approach. To Fix01. Track to FOS NDB, 348.0. Commence a 400fpm descent to 1,000ft MSL. When established on course tune ADF to BLK NDB, 328.0. When bearing to station is 220deg (96deg right) make a right turn to 180deg…………………………………………. To Fix02. Commence a 300fpm descent to 800ft MSL. When bearing to station is 256deg (76deg right) make a right turn to runway Hdg 257deg………………………….. To runway. Maintain Hdg 257deg for visual approach to runway………………………… Land Blackbushe Rwy 26. Length – 4,964ft. Width – 151ft. Surface – Asphalt. Missed approach. Climb on runway heading to 2000 ft. Turn Right for Woodley NDB 352. At Woodley NDB turn Right for Fairoaks NDB 348. At Fairoaks NDB continue on course past the beacon for two minutes, Then make a Rate 1 Left turn until you are heading back towards Fairoaks NDB again. Once past Fairoaks NDB for the second time, descend to 800 ft. Tune the ADF to Blackbushe NDB 328 and head straight for the beacon. Runway 26 should appear to the right of the beacon. |  |  |  |  | 313deg 200deg 174deg 170deg 145deg 162deg 135deg 118deg 128deg 124deg 180deg 257deg | 4.4nm 42nm 48.1nm 41.6nm 62.5nm 25.3nm 33.6nm 22.6nm 32.3nm 5.5nm 3.8nm 3.8nm | 00+02 00+17 00+19 00+16 00+24 00+10 00+13 00+09 00+13 00+03 00+02 00+02 |
| Flight No.196-01-05 | Arrival Airport Elev. – 328ft |  |  | Estimated totals for this flight>>> |  |  | 326nm | 02+11 |