# Leg 3  196-01-08 Reykjavik to Narsarsuaq	Time of Day: Dawn		Season: Winter

Please set preferred weather – with westerly winds.

### REYKJAVIK Iceland BIRK

Tune ADF to Reykjavik RK NDB freq. 355

Take off Runway 32

Continue on runway heading to Reykjavik RK NDB

S/C for Keflavik NDB freq. 392

Cleared initially to 4000 ft.

Reqd. track: 264 deg.

### KEFLAVIK NDB 392      				N63º 59’  W022º 44’

S/C for Prins Christian Sund NDB freq. 372     	N60º 04’  W043º 10’

### Initial Great Circle Track: 279º

Work out:  Change of Longitude and Mean Latitude

and from these, calculate the Conversion Angle using the ABAC.

Then choose to fly either the Rhumb Line Course:   Initial G.C. Track minus the C.A.

or fly the G.C. route, reducing the heading by twice the C.A. over the flight.

Or: - Take the easy ‘route’ and use the following figures which were produced using the E6-B computer and have been checked and tested.

Initial GC heading = 279deg.

Final GC heading = 269deg.

Rhumb Line Heading = 274deg.

PRINS CHRISTIAN SUND Greenland NDB 372

S/C for Nanortalik NDB freq. 270

Reqd Track: 307 deg.

### NANORTALIK NDB 270

IF YOU ARE FLYING IN VMC, clear of cloud and within sight of the ground, and with no forecast of deteriorating conditions, you may be cleared direct to

Bluie West One at a height of  5000 ft until the fjord is in sight,

when you may descend and make a visual approach to Runway 7.

Track from Nanortalik NDB to Narsarsuak NDB freq. 359 is 028 deg.

IN ALL OTHER CONDITIONS:  S/C for Simiutaq NDB freq. 279

Reqd. Track: 343 deg.

Descend to 4000 ft.

#### SIMIUTAQ NDB  279

Set course for Narsaq NDB  (Ident.NS; Freq. 404)

Required Track: 085 deg.

After you have passed Simiutaq NDB, descend to 1000 ft.

#### NARSAQ NDB  404

At Narsaq NDB maintain heading and descend to 500 ft., or lower if necessary to break clear of cloud, down to the Company Minimum; to fly a visual approach to Bluie West One.

Visual Approach to Bluie West One, in poor visibility.

Narsaq NDB  lies on the North shore of the entrance to the fjord.

It is essential in bad visibility to keep the North, or Left shore in sight at all times.

Follow the coastline, keeping just to the seaward side of the shore, in a slow cruising configuration.

5 miles into the fjord, ignore the inlet to the left. Bear right to pick up the main coastline, which continues in its original Easterly direction.

Tune the Radio Compass to Narsarsuaq NDB (Ident. NA; Freq. 359),

but DO NOT head directly for it at this point.

After a further 6 miles, the coastline bends gently to the left, and after another 8 miles turns more sharply until it lies on a heading of 020 deg. Set landing configuration.

At this point, continue flying the heading 020 deg., and watch the needle of the Radio Compass.  When it indicates a bearing of  050deg to the right, turn Right onto a heading of 073 deg. The Radio Compass should now indicate straight ahead.

Be prepared to anticipate this, so that as you roll out onto the heading, the runway should appear dead ahead.

Note that the NDB is not on the runway centreline, but lies to the left of it.

In real life, as the runway slopes upwards to the East – towards the rising ground of the glacier, landings are normally made to the East, and take-offs to the West.

(Note that in FS2002 there is a low escarpment at the edge of the airfield, so if you are making a really low approach, under the cloud base, you may have to climb to see the runway.)

Missed Approach.

This is not a good place to miss your approach in poor visibility, surrounded as you are by high ground, and if the weather is so bad that a safe approach is impossible, the preferred option would be to return to Goose.

However, if strong headwinds make this impossible, as a final resort you could attempt a let down to sea level over the main area of the fjord using the Narsarsuak NDB.

If the visual approach has to be aborted, make a maximum rate-of-climb to 3500 ft and head for the Narsarsuak NDB.

Overhead the NDB continue on an Easterly heading for two minutes, and make a rate 1 turn to the left, to head back to the NDB.

Overhead the NDB, turn onto a heading of 220 deg., and start a steep but slow descent, at 90 kts., with full flap and undercarriage down, to 500 ft, or to within sight of the sea. Ease the rate of descent as you approach sea level. (You don’t have to worry about passengers’ ears popping, as this is a freight flight - on the return journey you will be ferrying aircrew home, but they will appreciate the problem, and you getting them down in one piece.)

Turn right onto a heading of 020 deg., and watch the Radio Compass needle until it shows a bearing of 050 deg. Right.

Turn Right onto a heading of 070 deg., which should centre the needle, and fly towards the NDB. This should take you over the end of the runway.