# Leg 4  196-01-09 Narsarsuaq to Goose Bay.	Time of Day: Dawn		Season: Winter

Please set preferred weather – with westerly winds.

### BLUIE WEST ONE (Narsarsuaq) BGBW

### Tune ASDF to Narsaq NS NDB freq. 404 but DO NOT fly directly to the beacon.

Take-off is always to the West, from the downhill Runway 25.

After take-off turn left to a heading of 220 deg.

Cleared initially to 4000 ft.

When the ADF shows Narsaq NS NDB to be 56 deg. to the right of heading, turn right onto the 276 QDM to the beacon.

### NARSAQ NDB freq. 404

S/C for Simiutaq NDB freq 279

Required track 266 deg.

Cleared to climb to 8000 ft.

### SIMIUTAQ NDB freq 279 		(Lat 60º 41’  Long 46º 36’)

### S/C for Rigolet NDB freq. 396	(Lat 54º 11’  Long 58º 26’)

Work out Conversion Angle from Change of Longitude and Mean Latitude, and

choose either Great Circle Route:   initial GC course 258 deg. or Rhumb line Course.

RIGOLET NDB freq 396

S/C for Goose (Goose Bay) NDB freq 257

Required track: 262 deg.

Descend to 4000 ft.

### Approach to GOOSE BAY Runway 26

If weather conditions are suitable, you can make a visual descent and approach to Runway 26.

For an instrument let down:

Over Goose NDB at 4000 ft

Make a standard rate 180deg turn to 083deg.

Commence a 500fpm descent to 2000 ft

Fly the 083deg. heading for four minutes

Make a standard rate 180deg right turn to runway heading 263deg to head back to the NDB. Descend to 500ft MSL.

Over Goose NDB at 500ft.

Descend on a heading of 263 deg until runway lights are seen, or Company Minimum altitude is reached.

For a missed approach, climb on runway heading to 4000 ft.

Turn left towards Goose NDB and repeat descent pattern.