| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy -26 | Init. Hdg – 204deg | Init. Alt – 8,000ft |  | Apt Elev. – 157ft |  |  |  |
| Goose Bay (CYYR) Canada To Presque Isle (83B) USA | Departure. To Fix 02. After take off turn left to 204deg and intercept the 239deg bearing OB (145deg left) from YR NDB, 257.0 Climb to 8,000ft MSL………………………………… Enroute. To YGV NDB, 344.0………………………………………………………………………….. To PN NDB, 360.0……………………………………………………………………………. To YVB NDB, 243.0………………………………………………………………………….. To YSL NDB, 404.0. Commence a 500fpm descent to 4,000ft MSL…………………… Approach. To PQ NDB, 278.0. Commence a 400fpm descent to 2,000ft MSL…………………….. To runway. When overhead PQ NDB make a left turn to 186deg and fly heading for one minute. Commence a 400fpm descent to 1,500ft MSL. Make a right 45/180/45deg procedure turn to runway heading 006deg. Commence a 400fpm descent to 1,000ft MSL. When overhead PQ NDB commence descent for visual approach to runway….. Land Presque Isle Rwy 1. Length – 7,433ft. Width – 150ft. Surface – Asphalt. Missed approach. Climb on Runway heading to 2000 ft. MSL. Make a right turn back towards PQ NDB and repeat approach. |  |  |  |  | 204deg 239deg 252deg 226deg 261deg 210deg Final Hdg 006deg | 7.6nm 213.0nm 37.0nm 113.9nm 107.5nm 36.3nm 13.7nm | 00+04 01+22 00+14 00+43 00+44 00+15 00+07 |
| Flight No.196-01-10 | Arrival Airport Elev. – 498ft |  |  | Estimated totals for this flight>>> |  |  | 530nm | 03+29 |