1000-01-01 Blackbushe to Nice.

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy - 26 | Init. Hdg - 257 | Init. Alt - 1200 |  | Apt Elev. – 328ft |  |  |  |
| Blackbushe (EGLK) Hampshire, UK To Nice (LFMN) Cote D’Azur France | Tune ADF TO SHM NDB 332.00. Departure. To SHM NDB 332.0. After take off maintain runway heading to 1200ft. Then turn left to intercept 145° bearing to station. Commence climb to 7000ft MSL. Direct to NDB………………………………………………………………………………………. En-Route. To DSA NDB 351.0: Turn slight left to 141°. Continue on SHM NDB outbound bearing 141° (Bearing to station=321°). When signal fades, tune ADF to DSA NDB 351.0. Maintain heading until signal received. Direct to NDB……………………… To TH NDB 302.0: Turn slight right to 146°. Continue on DSA NDB outbound bearing 146° (Bearing to station=326°). When signal fades, tune ADF to TH NDB 302.0. Maintain heading until signal received. Direct to NDB………………………. To AX NDB 417.0: Turn slight left to 141°. Continue on TH NDB outbound bearing 141° (Bearing to station-321°). When signal fades, tune ADF to AX NDB 417.0. Maintain heading until signal received. Direct to NDB……………………………….. To MO NDB 400.0: Turn right to 153°. Continue on AX NDB outbound bearing 153° (Bearing to station=333°). When signal fades, tune ADF to MO NDB 400.0. Maintain heading until signal received. Direct to NDB……………………………… To MTL NDB 362.0: Turn right to 183. Continue on MO NDB Outbound bearing 183 (Bearing to station = 003). Climb to 8000ft MSL. When signal fades, tune ADF to MTL NDB 362.0 Maintain heading until signal received. Direct to NDB…. To ALM NDB 430.0: Turn left 159. Continue on MTL NDB outbound bearing 159 (Bearing to station = 339). Commence 300fpm descent to 7000ft MSL. When signal fades, tune ADF to ALM NDB 430.0 Maintain heading until signal received. Direct to NDB ……………………………………………………………………………. To LU NDB 378.0: turn left to 099. Continue on ALM NDB outbound bearing 099 (Bearing to station = 279). When signal fades, tune ADF to LU NDB 378.0 Maintain heading until signal received. Direct to NDB…………………….. ……… Approach. To Fix 02: Continue on LU NDB outbound bearing 100. (Bearing to station = 280°) Start timer. When established on course, tune ADF to VNA NDB 326.0 Maintain heading until bearing to station = 040 Waypoint reached after approx thirteen minutes……………………………………………………………………….. To Fix 03. Circle to land. Turn left to 040. Start timer. Continue on VNA NDB 326.0 Bearing to station = 040 Commence 300fpm descent to 1300ft. Slow to 120Kts. When established on course, tune ADF to LEZ NDB, 399.0. Waypoint reached when bearing to station is 300°. Approx eleven minutes………………… To Fix 04. Turn left to 300 inbound LEZ NDB 399.0. Start timer. When established on course tune ADF to NC NDB 338.0. Waypoint reached when bearing to station = 240°. Anticipate turn after Approx six minutes. Visual with ground must be established…………………………………………………………..... To Fix 05 Turn left to 240 Start timer. Maintain heading for approx two minutes while looking right for point of peninsular. Waypoint reached when level with point of peninsular……………………………………………………………………………… To Fix 06 Turn right to 305. Head towards the bay. Look left for runway. When appropriate turn left to runway heading 226. Visual approach to runway…………. Land Rwy 22R (226deg). Length - 8425ft, width - 148ft, surface – Asphalt … …… DO NOT OVERFLY THE CAPE AND CITY OF ANTIBES |  |  |  |  | 145 141 146 141 153 183 159 099 100 040 300 240 305 226 | 39.0nm 78.0nm 84.0nm 68.0nm 111.0nm 103.0nm 70.0nm 51.0nm 34.0nm 25.0nm 12.0nm 3.0nm 3.0nm 3.0nm | 00+16 00+30 00+32 00+26 00+43 00+39 00+27 00+20 00+13 00+12 00+06 00+01 00+01 00+02 |
| Flight No. 1000-01-01 | Arrival Airport Elev. - 13ft |  |  | Estimated totals for flight>>> |  |  | 686nm | 04+29 |