Nice to Malta

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy – 22R | Init. Hdg - 223 | Init. Alt – 1000ft |  | Apt Elev. – 13ft |  |  |  |
| Nice (LFMN) CoteD’Azur, France To Luqa (LMML) MALTA | Tune ADF to NC NDB 338.0. Departure To NC NDB 338.0. After take off climb to 1000ft. Direct to NDB…………………. To Fix 01. Turn left to 105°. Continue on outbound bearing 105° (Bearing to station=285°). Commence climb to 7000ft MSL. When established on heading, tune ADF to LEZ NDB 399.0. Waypoint reached when bearing to station reads 319°……………………………………………………………………………………… ### En-Route To CV NDB 375.0. Turn right to137. Continue on LEZ NDB outbound bearing 137 (Bearing to station = 317). When signal fades, tune ADF to CV NDB 375.0 Maintain heading until signal received. Direct to NDB …………………………….. To SZA NDB 349.5. Continue on CV NDB outbound bearing 146 (Bearing to station = 326). When signal fades, tune ADF to SZA NDB 349.5 Maintain heading until signal received. Direct to NDB …………………………………………………. To SME NDB 357.0. Turn right to 176 Continue on SZA NDB outbound bearing 176 (Bearing to station = 356). When signal fades, tune ADF to SME NDB 357.0. Direct to NDB …………………………………………………………………………… To ARB NDB 289.0. Turn slight left to 172, Start timer. Continue on SME NDB outbound bearing 172. After ten minutes tune ADF To ARB NDB 289.0. Intercept ARB NDB Inbound Bearing 172°. Direct to NDB………………………….. To CAR NDB 402.0. Turn slight right to 189. Climb to 8000ft MSL. Direct to NDB……………………………………………………………………………………….. To TRP NDB 317.5. Turn left to 116. Continue on CAR NDB outbound bearing 116. (Bearing to station = 296) Descend to 7000ft MSL. Maintain heading until signal fades. Tune ADF to TRP NDB 402.0. When signal received intercept inbound bearing 116. Direct to NDB….. …………………………………………… To Fix 02. Continue on TRP NDB outbound bearing 143 (Bearing to station = 323). Start timer When signal fades, tune ADF to GZO 320.0 and intercept 143 bearing to station when signal received. Waypoint after forty minutes………… ### Approach To GZO NDB 320.0 Maintain course 143. Commence 300fpm descent To 1800ft MSL. Direct to NDB……………………………………………………………………… To fix 03. Turn left to 125. Start timer. Tune ADF to MLT NDB 395.0. Waypoint at Interception of 134 inbound bearing to station. Reached after approx four minutes…………………………………………………………………………………….. To runway. Turn right to runway heading 134 for visual approach to runway……. Land Luqa Rwy 14. Length - 11528ft. Width - 197ft. Surface – Asphalt. |  |  |  |  | 223 105 137 146 176 172 189 116 143 143 125 134 | 6.0nm 22.0nm 75.0nm 47.0nm 62.0nm 59.0nm 50.0nm 157.0nm 104.0nm 35.0nm 9.0nm 8.0nm | 00+03 00+09 00+29 00+18 00+24 00+23 00+19 01+00 00+40 00+17 00+04 00+04 |
| Flight No. 1000-01-02 | Arrival Airport Elev. - 298ft |  |  | Estimated totals for flight>>> |  |  | 634.0 | 04+10 |