Malta to Benina

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy - 14 | Init. Hdg - 135° | Init. Alt – 1000ft |  | Apt Elev. – 298 ft |  |  |  |
| Luqa (LMML) MALTA To Benina (HLLB) (Benghazi) Libya | Tune ADF to MLT NDB 395.0. Departure To MLT NDB 395.0. Maintain runway heading. Climb to 1,000ft MSL. Direct to NDB………………………………………………………………………………………. En-Route T0 Fix 01. Turn left to 126. Start timer. Commence climb to 7,000ft. Intercept MLT NDB 126 bearing outbound (Bearing to station = 306). When signal fades maintain heading and tune ADF to BNA NDB 342.0. When signal received, intercept 126° bearing to station. Waypoint reached after 2 Hour 05 Minutes…… Approach To Fix 02. Turn slight left to 123°. Start Timer. Tune ADF to BDR NDB 278.0. Maintain heading until signal received. Intercept 123° bearing to station. Descend to 1,800ft MSL. Waypoint after fifteen minutes……………………………………… To BNR NDB 278.0. Maintain heading. Descend to 1,200ft MSL. Slow to 120kts. Direct to NDB……………………………………………………………………………. To Rwy. Turn right to 147° for visual approach to runway…………………………. Land Rwy 15 L. (149 deg). Length -11810 ft. Width - 148ft. Surface – Asphalt. |  |  |  |  | 135 126 123 123 147 | 4.0nm 310.0nm 37.0nm 11.0nm 4.0nm | 00+02 02+00 00+15 00+06 00+02 |
| Flight No. 1000-01-03 | Arrival Airport Elev. - 433 ft |  |  | Estimated totals for flight>>> |  |  | 366nm | 02+25 |