1000-01-04 Benina to Khartoum. Via Shark El Oweinat, Egypt.

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy – 15L | Init. Hdg - 128deg | Init. Alt - 7,000ft |  | Apt Elev. - 433ft |  |  |  |
| Benina (HLLB) (Benghazi) Libya To Shark El Oweinat (HEOW) Egypt | Tune ADF to BS NDB 257.0. Departure. To BS NDB 257.0. After take off maintain Rwy heading. Commence climb to 7,000ft MSL. Direct to NDB………………………………………………….............. To Fix 01. Turn right to 175°. Tune ADF to BNA NDB 342.0. Waypoint when bearing to station = 342°……………………………………………………………… En-Route. To GAL NDB 273.0. Turn left to bearing 162° (Bearing to station = 342°). Maintain heading until signal fades, then tune ADF to XY NDB 325.0. When signal received, direct to NDB……………………………………………………………… To GS NDB 305.0. Turn left to 137° (Bearing to station = 317°). When established on course tune ADF to GS NDB 305.0. Direct to NDB………………………………. To Fix 02. Turn left to 129° (Bearing to station = 309°). Maintain heading until signal fades, then tune ADF to OWT NDB 301.0. Waypoint when signal received. (Note - Signal will fade after approximately 30 minutes leaving almost 350nm of this leg to be navigated by DED. Ensure accuracy of heading before signal fades.) Approach. To OWT NDB 301.0. Descend to 2,500ft MSL. Slow to 120kts. Direct to NDB…… To Rwy. Turn right to 160°. Start timer. Fly heading for three minutes. (Fix 03) Turn right to 270°. Observe ADF bearing to station. As heading nears 009° turn right to runway heading 007° (Fix 04). Visual approach to runway……. Land Rwy 1 (007deg). Length – 11,521 ft. Width – 148 ft. Surface = Asphalt. |  |  |  |  | 128 175 128 317 129 129 Final Hdg 007 | 06.0nm 07.0nm 193.0nm 84.0nm 417.0nm 38.0nm 14.0nm | 00+03 00+03 01+15 00+32 02+40 00+18 00+07 |
| Flight No. 1000-01-04 | Arrival Airport Elev. - 859 ft |  |  | Estimated totals for flight>>> |  |  | 759nm | 04+58 |

Refuel.

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy – 1 | Init. Hdg - 07deg | Init. Alt - 2,500ft |  | Apt Elev. - 859ft |  |  |  |
| Shark El Oweinat (HEOW) Egypt To Khartoum (HSSS) Sudan | Tune ADF to OWT NDB 301.0. Departure. To Fix 05. Start timer. After take off maintain runway heading. Commence climb to 7,000ft MSL. Waypoint after two minutes………………………………………… To Fix 06/07. Make a standard rate right turn to 170°. Waypoint when bearing to station =333°……………………………………………………………………………. En-Route. To DO NDB 310.0. Turn left to bearing 153° (Bearing to station = 333°). Maintain heading until signal fades, then tune ADF to DO NDB 310.0. When signal received, direct to NDB………………………………………………………………… To DBA NDB 650.0.0. Turn slight left to 154° (Bearing to station = 334°). When signal fades tune ADF to DBA NDB 650.0. Direct to NDB………………………… To Fix 08. Turn left to 146° (Bearing to station = 326°). Start timer. Maintain heading until signal fades, then tune ADF to KH NDB 343.0. When signal received intercept 146° bearing to station. Waypoint after fifty minutes………………….. Approach. To KH NDB 343.0. Maintain heading 146°. Descend to 3,500ft MSL. Slow to 120kts. Direct to NDB…………………………………………………………………. To Rwy. Turn right to 200°. Fly heading for five minutes. (Fix 09) Turn left to 090°. Observe ADF bearing to station. As heading nears 357° turn left to runway heading 358°. Visual approach to runway………. Land Rwy 36 (358deg). Length – 9,795 ft. Width – 148 ft. Surface = Asphalt. |  |  |  |  | 007 170 153 154 146 146 Final Hdg 358 | 4.0nm 07.0nm 222.0nm 73.0nm 130.0nm 40.0nm 20.0nm | 00+02 00+03 01+25 00+28 00+50 00+15 00+10 |
| Flight No. 1000-01-04 | Arrival Airport Elev. - 1,259 ft |  |  | Estimated totals for flight>>> |  |  | 496nm | 03+13 |