Day one, morning -Depart Bankstown in the morning in time to have lunch at Merimbula.

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 11C – 07:30L | Init. Hdg: 150deg |  | Init. Alt: 3500ft | Apt Elev: 26ft |  |  |  |
| Bankstown (YSBK), NSW, Australia To Merimbula (YMER), NSW, Australia | Departure. To Fix 02. After take off turn right Hdg 150deg and intercept the 195deg radial OB from SY VOR 112.10 (here we pick up the Low Altitude Airway H68). Climb to 3,500ft MSL…. En Route. To WOL NDB, 239.0 ………………………………………………………………………….. To NWA NDB, 359.0 …………………………………………………………………………. To Fix 03 (Low Altitude Airway H20) Tune Nav1 to MER DME, 117.70. When DME reads 25nm commence a 400fpm descent to 1,200ft MSL. Waypoint reached when DME reads 10nm……………………………………………………………………………………… Approach. To Fix 04. On station passage turn left to 159deg. Commence a 400fpm descent to 700ft MSL. Slow to 95kts. Waypoint reached when ADF bearing to station reads 199deg (40deg right)……………………………………………………………………………………… To runway. Turn right to runway Hdg 201deg. Visual approach to runway………………. Land Merimbula Rwy 21. Length – 5,254ft. Width – 98ft. Surface – Asphalt. Missed approach. Climb straight ahead to 1,200ft MSL. Then turn left and fly direct to MER NDB and hold. |  |  |  |  | 150deg 195deg 196deg 181deg 159deg 201deg | 12.5nm 30.5nm 27.0nm 111.6nm 4.7nm 5.0nm | 00+06 00+12 00+11 00+46 00+03 00+03 |
| Flight No:- 809-01-01 | Arrival Airport Elev: 6ft |  | Estimated totals for this flight>>> |  |  |  | 192nm | 01+21 |

Day one, afternoon – After a Lunch Stop at Merimbula we continue on to Flinders Island.

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 21 – 14:30L | Init. Hdg: 177deg | Init. Alt: 3,500ft |  | Apt Elev: 7ft |  |  |  |
| Merimbula (YMER), NSW, Australia To Flinders Island (YFLI), Tasmania, Australia | Departure. To MCO NDB, 338.0. After take off turn left Hdg 177deg. Climb to 3,500ft MSL. Fly direct to NDB…………………………………………………………………………………….. Enroute. To FLI NDB, 338.0. On station passage MCO NDB turn right to the 194deg bearing OB. Climb to 4,500ft MSL. When signal fades maintain Hdg. Dead Reckoning until FLI NDB received. Five minutes after FLI NDB is received commence a 400fpm descent to 1.500ft MSL, slow to 120kts……………………………………………………………………………. Approach. To runway. On station passage FLI NDB turn left to 160deg and fly heading for one minute, Commence a 400fpm descent to 800ft MSL. Turn left to 130deg and fly heading for one minute. Make a left 180deg turn to runway heading 310deg. The runway is almost impossible to see from a distance but there is a large red and white radio mast located to the left of the runway which will act as a visual guide to the airfield…………………………………………………………………………………………….. Land Flinders Isl. Rwy 32. Length – 5,642ft. Width – 98ft. Surface – Oil treated. Missed approach. Climb straight ahead to 800ft MSL, then climbing left turn to 1,500ft MSL. Return to FLI NDB and repeat approach. |  |  |  |  | 177deg 194deg Final Hdg 310deg | 42.8nm 170nm 8.6nm | 00+18 01+08 00+05 |
| Flight No:- 809-01-02 | Arrival Airport Elev: 32ft |  |  | Estimated totals for this flight>>> |  |  | 221nm | 01+31 |

Day Two – We leave about midday and head for Hobart, sometimes called ‘the jewel of the South’.

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 32 – 12:00L | Init. Hdg: 130deg |  | Init. Alt: 3,500ft | Apt Elev: 32ft |  |  |  |
| Flinders Island (YFLI), Tasmania, Australia To Hobart (YMHB), Tasmania, Australia | Departure. After take off make a left 180deg turn to Hdg 130deg and intercept the 157deg bearing OB from FLI NDB, 296.0. Climb to 3,500ft MSL……………………………………………… Enroute. To STH NDB, 392.0 …………………………………………………………………………… To HB VOR/DME, 112.70. Climb to 4,500ft MSL. When DME reads 12nm commence a 400fpm descent to 2,500ft MSL……………………………………..................................... Approach. To ISEC hb080. On station passage HB VOR turn left to Hdg 120deg. Commence a 400fpm descent to 1,500ft MSL. Slow to 120kts. Maintain Hdg until DME reads 8.0nm…………………………………………………………………………………………….. To runway. Make a right 45/180/45deg procedure turn and intercept the ILS, 109.90. Beware, this is a backcourse. Alternatively, fly a VOR approach………………………. Land Hobart Rwy 30. Length – 7,382ft. Width – 148ft. Surface – Asphalt. Missed Approach. Climb straight ahead to 1,500ft MSL, then climbing left turn to 2,500ft MSL. Fly to HB VOR. Repeat approach. |  |  |  |  | 130deg 157deg 185deg 120deg 300deg | 8.5nm 71.6nm 96.7nm 8.0nm 13.7nm | 00+04 00+29 00+39 00+04 00+08 |
| Flight No:- 809-01-03 | Arrival Airport Elev: 9ft |  | Estimated totals for this flight>>> |  |  |  | 199nm | 01+25 |

Day Four – Now we go to Strahan, after a tour over some of Tasmania’s central wilderness.

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 30 – 12:30L | Init. Hdg: 301deg |  | Init. Alt: 6500ft | Apt Elev: 9ft |  |  |  |
| Hobart (YMHB), Tasmania, Australia To Strahan, (YSRN), Tasmania, Australia | Departure. To HB NDB, 362.0. Climb to 6,500ft MSL......................................................................... Enroute. To Salem ISEC. Low altitude airway W710. On station passage HB NDB, turn right and intercept the 303deg radial OB from HB VOR, 112.70. Continue climb to 6,500ft MSL. Tune Nav2 to LT VOR, 112.60 and set OBS to 051deg. Waypoint is reached when Nav1 DME reads 80nm and Nav2 needle centers…………………………………………………... To SRN NDB, 257.0. Tune Nav1 to LT VOR, 112.60. When DME reads 65nm Commence a 400fpm descent to 1,700ft MSL……………………………………………….. Approach. To runway. On station passage SRN NDB, turn left to 209deg. Commence a 400fpm descent to 1,000ft MSL. Fly Hdg for one minute. Turn left to 179deg and fly Hdg for two minutes. Make a left 180deg turn to runway Hdg 359deg for visual approach to runway…………. Land Strahan Rwy 36. Length – 4,001ft. Width – 98ft. Surface – Asphalt. Missed approach. Climb straight ahead to 1,700ft. Turn left and fly to SRN NDB. Repeat approach. |  |  |  |  | 301deg 303deg 236deg Final Hdg 359deg | 2.6nm 77.0nm 47.5nm 12.0nm | 00+01 00+31 00+19 00+06 |
| Flight No:- 809-01-04 | Arrival Airport Elev: 66ft |  | Estimated totals for this flight>>> |  |  |  | 139nm | 00+58 |

Day Five – After a morning river cruise we move on to King Island.

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 36 – 11:45L | Init. Hdg: 360deg |  | Init. Alt: 4500ft | Apt Elev: 66ft |  |  |  |
| Strahan (YSRN), Tasmania, Australia To King Island (YKII), Tasmania, Australia | Departure. To WYY VOR, 115.80. After take off turn right to 003deg. Climb to 5,500ft MSL. Note. No DME at WYY VOR. Note time of passing station……………………………………………. En Route. To Fix 02. After approx 32min. flying time commence a 400fpm descent to 1,500ft MSL. Slow to 120kts………………………………………………………………………………….. To Fix 03. Tune ADF to KII NDB, 332.0. Maintain Hdg until ADF bearing to station reads 279deg (19deg left). Waypoint reached at this point………………………………………. Approach. To runway. On station passage turn left to runway Hdg 283deg. Commence a 300fpm descent to 500ft MSL. Visual approach to runway………………………………………….. Land King Island Rwy 28. Length – 5,193ft. Width – 98ft. Surface – Oil Treated. Missed approach. Climb straight ahead to 500ft MSL, then climbing left turn to 1,500ft MSL. Fly to KII NDB. Fly OB on Hdg 103deg for five minutes. Make a left 180deg turn to runway Hdg 283deg and repeat approach. |  |  |  |  | 003deg 298deg 298deg 283deg | 72.4nm 79.9nm 18nm 10nm | 00+30 00+32 00+09 00+07 |
| Flight No:- 809-01-05 | Arrival Airport Elev: 131ft |  | Estimated totals for this flight>>> |  |  |  | 180nm | 01+17 |

Day Seven – Kangaroo Island beckons. It is home to colonies of fairy penguins, sea lions, wallabies, koalas and many more Australian creatures.

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 28 – 08:30L | Init. Hdg: 283deg | Init. Alt: 4500ft |  | Apt Elev: 131ft |  |  |  |
| King Island (YKII), Tasmania, Australia To Kingscote (YKSC), South Australia, Australia | Departure. To Fix 01. After take off maintain Rwy Hdg and intercept the 300deg bearing OB from KII NDB, 332.0. Climb to 4,500ft MSL………………………………………………………… En Route. To POD NDB, 212.0. Continue climb to 4,500ft MSL………………………………………… To MTG VOR, 117.00 …………………………………………………………………………… To LAKES ISEC Tune Nav1 to AD VOR, 116.40. Tune Nav2 to TBD VOR, 116.60. Set Nav2 OBS to 042deg. Waypoint reached when Nav1 DME reads 45nm and Nav2 needle centers…………………………………………………………………………………………….. Approach. To KSC NDB, 215.0. Set Nav1 OBS to 027deg. When needle approaches center commence a 400fpm descent to 1,500ft MSL………………………………………………… To runway. On station passage turn right to 343deg and fly Hdg for one minute. Commence a 300fpm descent to 500ft MSL. Turn right to 013deg and fly Hdg for two minutes. Make a right 180deg turn to runway Hdg 193deg. Visual approach to runway…………… Land Kingscote Rwy 19. Length – 4,609ft. Width – 98ft. Surface – Asphalt. Missed approach. Climb straight ahead to 500ft MSL, then climbing right turn to 1,500ft MSL. Fly to KSC NDB and repeat approach. |  |  |  |  | 283deg 300deg 306deg 318deg 256deg 193deg | 3.5nm 143.0nm 46.9nm 155.6nm 73.7nm 13.0nm | 00+02 00+58 00+20 01+21 00+30 00+07 |
| Flight No:- 809-01-06 | Arrival Airport Elev: 23 |  |  | Estimated totals for this flight>>> |  |  | 436nm | 02+58 |

Day Nine – Arrive in Ballerat at around lunchtime and return to the gold rush days.

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy:19 – 09:30L | Init. Hdg: 065deg | Init. Alt: 5,500ft |  | Apt Elev: 22ft |  |  |  |
| Kingscote (YKSC), South Australia, Australia To Ballarat (YBLT), Victoria, Australia | Departure. To Fix 02. After take off turn left to 065deg and intercept the 076deg bearing OB from KSC NDB, 215.0. Climb to 5,500ft MSL………………………………………………………. En Route. To Lakes ISEC. Tune Nav1 to AD VOR, 116.40. Set OBS to 319deg. Continue climb to 5,500ft MSL. Waypoint reached when needle centers………………………………………. To MTG VOR, 117.00. On station passage Lakes ISEC turn right to the 138deg radial OB from AD VOR. When DME reads 95nm Tune Nav1 to MTG VOR and fly direct to station…………………………………………………………………………………………….. To Fix 03. Note time of passing station. On station passage MTG VOR, turn left to the 079deg radial OB. After forty minutes flying time Tune Nav1 to ML VOR, 114.10 and track. Tune ADF to BLT NDB, 239.0. Waypoint reached when DME reads 73nm………. Approach. To runway. Commence a 500fpm descent to 2,500ft MSL. Track to BLT NDB until runway visual. CAUTION- Beware the substantial ridge on the approach to Rwy 5.... Land Ballerat Rwy 5. Length – 4,148ft. Width – 98ft. Surface – Asphalt. Missed approach. Climb straight ahead to 2,500ft MSL. Turn right and fly to BLT NDB. Continue OB on Hdg 231deg for two minutes. Make a right 45/180/45deg procedure turn to runway Hdg 051deg and repeat approach. |  |  |  |  | 065deg 076deg 138deg 079deg 051deg | 10.5nm 65.3nm 155.6nm 120.0nm 25.5nm | 00+05 00+26 01+01 00+47 00+13 |
| Flight No:- 809-01-07 | Arrival Airport Elev: 1,430ft |  |  | Estimated totals for this flight>>> |  |  | 377nm | 02+32 |

Day Ten, morning - Arrive in time for a sumptuous lunch in wine country.

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 5 – 10:30L | Init. Hdg: 50deg | Init. Alt: 5,500ft |  | Apt Elev: 1,430ft |  |  |  |
| Ballarat (YBLT), Victoria, Australia To Corowa (YCOR), New South Wales, Australia | Departure. To MNG VOR, 113.20. After take off turn to Hdg 050deg. Climb initially at 900fpm to 5,500ft MSL........................................................................................................................ En Route. To Fix 02. On station passage MNG VOR turn left to 035deg radial OB. Commence a 500fpm descent to 3,500ft MSL. Tune Nav2 to AY VOR, 115.60 and set OBS to 064deg. Waypoint reached when needle centers. DME should read 49nm………………………… Approach. To Fix 03. Track to COR NDB, 380.0. Commence a 400fpm descent to 1.200ft MSL. Maintain track until runway visible……………………………………………………………… To runway. Turn right to runway Hdg 043deg for visual approach………………………… Land Corowa Rwy 5. Length – 5,991ft. Width – 148ft. Surface – Asphalt. Missed approach. Climb straight ahead to 1,200ft MSL. Turn right and fly to COR NDB. Continue OB on Hdg 223deg for two minutes. Make a right 45/180/45deg procedure turn to runway Hdg 043deg and repeat approach. |  |  |  |  | 050deg 035deg 035deg 043deg | 77.0nm 53.0nm 22.7nm 1.1nm | 00+31 00+21 00+12 00+01 |
| Flight No. 809-01-08 | Arrival Airport Elev: 465ft |  |  | Estimated totals for this flight>>> |  |  | 154nm | 01+05 |

Day Ten, afternoon – Time for a fond farewell as we head back north to Sydney.

| From - To | Flight Description. "Allocated runways and related information may change when flying online or using Real Weather" |  |  |  |  | Course (Leg) | Distance (Leg) | ETE(leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy: 5 – 15:00L | Init. Hdg: 036deg | Init. Alt: 5,500ft |  | Apt Elev: 465ft |  |  |  |
| Corowa (YCOR), New South Wales, Australia To Bankstown (YSBK), New South Wales, Australia | Departure. To WG VOR, 115.00. After take off turn slight left to 036deg. Climb to 5,500ft MSL…… En Route. To YAS NDB, 335.0…………………………………………………………………………….. To BIK VOR, 116.80……………………………………………………………………………. Approach. To Fix 02. On station passage BIK VOR turn right to 053deg radial OB. Commence a 400fpm descent to 1,500ft MSL. Waypoint reached when DME reads 25nm……………. To Fix 03. Maintain Hdg and commence a 400fpm descent to 1.000ft MSL. Tune Nav1 to SY VOR, 112.10. Set OBS to 094deg. Waypoint is reached when needle centers…… To runway. On station passage turn right to runway Hdg 111deg. Commence a 300fpm descent to 500ft MSL. Monitor track to BK NDB, 416.0 until runway visible then visual approach to runway……………………………………………………………………………… Land Bankstown Rwy 11C. Length – 4,643. Width – 98ft. Surface – Asphalt. Missed approach. Climb straight ahead to 500ft MSL then climbing right turn to 1,000ft MSL and fly to BK NDB. Continue OB on Hdg 291deg for three minutes. Make a left 45/180/45 deg procedure turn to Rwy Hdg 111deg and repeat approach. |  |  |  |  | 036deg 064deg 042deg 053deg 052deg 111deg | 74.3nm 79.9nm 65.5nm 35.1nm 8.3nm 5.0nm | 00+30 00+32 00+26 00+14 00+05 00+03 |
| Flight No. 809-01-09 | Arrival Airport Elev: 26ft |  |  | Estimated totals for this flight>>> |  |  | 268nm | 01+50 |