A DC-3 World Rally 2003 Flight

Bodo to Kjaerstad.

This flight has been derived from a very popular series of DC-3 Airways charter flights, ‘The Norwegian Fjords Charters’. Thanks to DC-3 Airways CEO Ron Bushell who created the original charters.

The weather forecast is for moderate winds that will be very blustery at low levels, so an adequate supply of the appropriate type of paper bags should be made easily available for the passengers on this trip.

After departure you will turn to a south westerly direction to follow the coast, taking care to avoid the islands on the route.

The approach to Kjaerstad is a very scenic journey through several Fjords. You will then face a very blustery and interesting approach to the destination airport.

Best of luck and enjoy the flight.

"Begin this flight with all tanks full."

| From - To | Flight Description |  |  |  |  | Course (Leg) deg | Distance (Leg) nm | ETE (leg) HH+MM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dep. Rwy - 25 | Init. Hdg - 224deg. |  | Init. Alt-2,000ft | Rwy Elev - 39ft. |  |  |  |
| Bodo (ENBO), Norway. To Kjaerstad (ENMS), Norway. | Preliminary settings: Tune ADF to STT NDB, 317.0. Tune Nav1 to STO VOR/DME, 112.00 and set OBS to 192deg. Departure. To STT NDB, 317.0: After take off maintain runway heading to 1,000ft, then turn left to 224deg. Commence climb to 2,000ft. Direct to NDB……………………..... Enroute. To Fix 02: Turn slight left to 215deg bearing outbound from STT NDB. Tune Nav1 to STO VOR/DME, 112.00. Check OBS is set to 192deg. Waypoint reached when needle centers. DME will read approx. D35.0……………………. To Fix 03: Turn slight left to intercept STO VOR R-192 inbound and track to VOR. Waypoint reached at STO VOR D8.5………………………………………. To Fix 04: Turn left to intercept VFL VOR/DME, 117.00. (112.25 in FS2004) R-125 inbound. Slow to 120kts. Waypoint reached at VFL VOR D15.0. To Fix 05: Turn right to 142deg. set Nav1 OBS to 117deg. Waypoint reached when needle centers. VFL VOR DME will read approx D10.0………………….. To Fix 06: Turn left to intercept VFL VOR R-117 inbound. Waypoint will be reached at VFL VOR D4.0…………………………………………………………… Approach. To runway: Turn right to 171deg. Commence a 500fpm descent to 700ft. Keep well clear of the steep escarpment to your right. Visual approach to runway which is offset 10deg the left of your approach angle…………………………………….. Land Kjaerstad Rwy 16. Length – 2,874ft. Width – 98ft. Surface – Asphalt. Missed approach. Turn slight right to 170deg commence climb to 3500ft (600fpm min). Tune Nav1 to Rwy 34 ILS 110.70. At D3.0 turn right to intercept STO VOR/DME R-305 inbound. Direct to VOR. Turn right to STO R-012. Commence 500fpm descent to 2,000ft. At STO D8.5, turn right to 125deg and repeat approach. |  |  |  |  | 224 215 192 125 142 120 171 | 31.0 28.4 26.8 8.0 4.9 6.4 7.4 | 00+13 00+12 00+11 00+04 00+02 00+03 00+05 |
| Flight No. DC-3 WR 04 | Arrival Rwy Elev. - 236ft. |  | Estimated totals for this flight>>> |  |  |  | 114nm | 00+51 |