Air Niugini Port Moresby to Nadzab, Gorka, Mt Hagen, Kerema, to Port Moresby

FSN flight plan: AN-1   (AYPY-AYNZ-AYGA-AYMH-AYKM- AYPY)

Schedule effective June 1, 1973

Total distance 642 miles.

| Flight | NM | From | To | Description | ICAO |
| --- | --- | --- | --- | --- | --- |
| AN-1a | 221 | Port Moresby | Nadzab | Dpt Pt Moresby 322 Deg to KUB NDB 1662.0. Dial the AYNZ VOR –NZ- 113.9.while climbing to 11000 feet. Change course to 331 passing over KUB and DR to WP3 at 44.7 miles. Change course to 003 for the NZ VOR. Proceed to Nadzab land RW 09/27. Elevation 229ft | AYPY-AYNZ |
| AN-1b | 86 | Nadzab | Gorka | Dpt Nadzab 284 Deg. Dial in Gorka NDB GA 1725.0. Climb to 10000 ft. Proceed to Gorka. .Land RW 17 or 35. Elevation 5186ft. | AYNZ-AYGA |
| AN-1c | 66 | Gorka | Mount Hagen | Dpt Gorka 284 Deg for WP (GA1) while climbing to 10000ft dial in MH VOR 115.0 and turn left when VOR bears 250. Proceed to Mt. Hagen to land RW 12/30 Elevation 5360ft | AYGA-AYMH |
| AN-1d | 173 | Mount Hagen | Kerema | Dpt Mt Hagen 209 Deg Climbing to 10000ft to pass over WP (MH1) when 40 miles from MH VOR. Turn to 124 Deg and dial in the KRM VOR on 116.9 Proceed to Kerema to land RW 14/32 Elevation 9 ft. | AYMH-AYKM |
| AN-1e | 152 | Kerema | Port Moresby | Dpt Kerema 131 Deg and climb to 3000ft. Proceed direct Port Moresby to land RW14/32 Elevation 124ft. | AYKM-AYPY |

Estimated flying time 04:32