Air Niugini – Nadzab, Hoskins, Rabaul (Tokua) Buka, Kiriwina to Port Moresby

Flight plan: AN-6   (AYNZ—HKN-AYTK-AYBK-AYKA- AYPA)

Schedule effective June 1, 1973

Total distance: 1052 miles.

| Flight | NM | From | To | Description | ICAO |
| --- | --- | --- | --- | --- | --- |
| AN-6a | 235 | Nadzab | Hoskins | Dial in FIN VOR 116.7 Dpt Nadzab 086 Deg and climb to 9000ft. Pass over the FIN VOR and turn left to 060 for the 167 mile leg to Hoskins (HKN VOR 114.0). Land RW 12/30 Elevation 34 ft. | AYNZ -HKN |
| AN-6b | 136 | Hoskins | Rabaul (Tokua) | Dpt Hoskins 055 Deg. Dial in Rabaul VOR TOK 113.6. Climb to 9000 ft. Proceed to Rabaul. Land RW 10/28 Elevation 32ft. | HKN-AYTK |
| AN-6c | 153 | Rabaul (Tokua) | Buka | Dpt Rabaul 108 Deg and climb to 7000ft. Dial in BUK VOR 114.7 and proceed to Buka. Land RW 04/22 Elevation 6 ft. | AHTK-AYBK |
| AN-6d | 283 | Buka | Kiriwina | Dpt Buka 222 Deg Climbing to 6000ft DR Direct to Kiriwina Land RW 14/32 Elevation 19ft | AYBK-AYKA |
| AN-6e | 243 | Kiriwina | Port Moresby | Dpt Kiriwina 258 Deg and climb to 10000ft. DR until you receive the GUA VOR 116.5 then fly over it.Turn left to 247 Deg and continue climb to 14000 ft. Dial in PY VOR 117.0. Turn left to PY when it bears 221. You will pass south of the 13500ft Mt.Victoria to your north and the 11500ft Mt Kerma to the south. Proceed Port Moresby. Land RW14/32 Elevation 124ft. | AYKM-AYPY |

Estimated flying time 07:30