Air Niugini – Port Moresby (Jacksons) Tapini, Garaina, Woitape, Port Moresby.

Flight plan: AN-8   (AYPY-TAP-GNA-WTP-AYPY

Schedule effective June 1, 1973

Total distance: 215 miles.

| Flight | NM | From | To | Description | ICAO |
| --- | --- | --- | --- | --- | --- |
| AN-8a | 77 | Port Moresby | Tapini | Depart Port Moresby 322 Deg for KUB NDB 1662. Climb to 4500ft. At KUB turn to a base course of 028 Deg. You will be flying up a narrow canyon 24 miles to land at TAP. (Reference jpg AN8 KUB to TAP) use RW 09/27 Elevation 3149ft. This is a very tight area; highly recommend a fly over before attempting to land. DAY TIME ONLY. | AYPY-TAP |
| AN-8b | 38 | Tapini | Garaina | Dpt Tapini 059 Deg. DR, follow the canyon, to WP AP1 at 10.8 miles while climbing to 9000ft. (Reference jpg AN8_TAP to GNA) At AP1 turn to 018 Deg to WP AP2, 15 miles from AP1 and on the east slope of the Whamton Range. Turn left to 326 Deg. You will be approx 10.3 miles from Garaina. Land Garaina RW 13/31 Elevation 2499ft. DAY TIME ONLY. | TAP-GNA |
| AN-8c | 47 | Garaina | Woitape | Dpt Garaina 135 Deg and DR to WP - NA1 - at 24 miles while climbing to 7000ft. (Reference jpg AN8_GNA to WTP) At NA1 turn right to 202 Deg follow the canyon and DR to Woitape 23 miles from NA1. Land RW 15/33 Elevation 5160ft. DAY TIME ONLY. | GNA-WTP |
| AN-8d | 54 | Woitape | Port Moresby | Dial in PY VOR 117. Dpt 175 Deg and follow canyon down to Port Moresby -. You will turn right then left to clear the Mount Cameron Range There’s a 3100 ft hill SW of the entrance to the canyon you are coming out of. Be careful. (Reference AN8_WTP to AYPY) | WTP-AYPY |

Estimated flying time 01:33

Owing to the terrain around TAP, GNA, and WTP you will need to deviate from the baseline headings. It will be necessary to follow the canyons and work through the terrain in order to be at an acceptable approach altitude for these airstrips. Suppose you could fly direct then spiral down several thousand feet over the airport to the correct approach altitude; your call. Recommend a flyover  at TAP, GNA and WTP prior to landing.