Air Mail Night Flight description for FS2004

| Air Mail Night Flight No. AM – SLC – EKO |  |  |  | ALL beacon frequencies are 400.5 KHz. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | In order to hear the Morse identification from the beacon, you must ensure that the ADF sound is turned ‘On’. |  |  |  |
| Salt Lake City Intl, Salt Lake City, Utah. (KSLC) To Elko Regl, Elko, Nevada. (KEKO) |  |  |  | There are two ways to configure the .5 element of the ADF frequency. 1. By setting the frequency in the FSNavigator ADF box. 2. Tune the 400 element in the normal way. Hit the ‘a’ key four times, then hit the ‘+’ key repeatedly until .5 is displayed. |  |  |  |
|  |  |  |  | The flight must be flown at night and in VFR conditions. If ‘Real weather’ is selected and proves to be unacceptable then the flight must be abandoned. A suitable weather situation may be substituted. It is permissible to commence the flight at dusk to enjoy the early flight scenery and if flying westwards, you may have an opportunity to enjoy the sunsets. |  |  |  |
| KEKO. Elevation – 5,141ft. Runways available. 23 – 233deg. 05 – 053deg. 30 – 298deg. 12 – 118deg. |  | KSLC. Elevation – 4,225ft. Runways available. 35 – 345deg. 17 – 165deg. 34R – 340deg. 16L – 160deg. 34L – 340deg. 16R – 160deg. 32 – 319deg. 14 – 139deg. |  | Morse code ident. 1: · ─ ─ ─ ─ 2: · · ─ ─ ─ 3: · · · ─ ─ 4: · · · · ─ 5: · · · · · 6: ─ · · · · 7: ─ ─ · · · 8: ─ ─ ─ · · 9: ─ ─ ─ ─ · 0: ─ ─ ─ ─ ─ |  |  |  |
| To Waypoint | Waypoint Altitude MSL. (ft) |  | Minimum Enroute Altitude MEA. (ft) | Flight Notes | Hdg (deg) | Dist (nm) |  |
| 0 (zero) 1 (one) 2(two) 3(three) 4(four) | 4,252 4,246 6,595 7,127 7,276 |  | 6,500 6,500 8,500 10,500 10,500 | After take off, climb straight ahead until stabilized, then circle left until able to approach center of airfield at 342deg. Depart on Hdg 342deg and climb to 6,500ft MSL……………………………… Maintain altitude of 6,500ft MSL…………………………………….. Climb to 8,500ft MSL…………………………………………………… Climb to 10,500ft MSL…………………………………………………. Maintain altitude of 10,500ft MSL……………………………………. | 342 250 244 255 242 |  | 16 26 22 34 23 |
| To Waypoint | Waypoint Altitude MSL. (ft) |  | Minimum Enroute Altitude MEA. (ft) | Flight Notes | Hdg (deg) | Dist (nm) |  |
| 5(five) 6(six) 7(seven) KEKO | 9,036 10,606 7,217 * * * * |  | 12,500 12,500 10,500 * * * * | Climb to 12,500ft MSL………………………………………………….. Maintain altitude of 12,500ft MSL……………………………………. It is safe to descend to 10,500ft MSL……………………………….. Turn left to 226deg and commence descent to 8,500ft MSL. If still above altitude of 8,500ft MSL when airport is reached, enter a holding pattern and continue to descend until 8,500ft MSL is reached. Make own arrangement for approach and landing. There is a small beacon located on the North side of the airfield close to the start of Rwy 12 which will aid identification of airport. The area around the airport is quite flat with no major obstructions and Maneuvering for landing at 8,500ft MSL is safe. | 272 251 253 226 | 14 24 32 8 |  |
| Total mileage for flight >>> |  |  |  |  | 199 nm |  |  |