Air Mail Night Flight description for FS2004

| Air Mail Night Flight No. AM – EKO – RNO |  |  |  | ALL beacon frequencies are 400.5 KHz. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | In order to hear the Morse identification from the beacon, you must ensure that the ADF sound is turned ‘On’. |  |  |  |
| Elko Regl, Elko, Nevada. (KEKO) To Reno Tahoe Intl. Reno, Nevada. (KRNO) |  |  |  | There are two ways to configure the .5 element of the ADF frequency. 1. By setting the frequency in the FSNavigator ADF box. 2. Tune the 400 element in the normal way. Hit the ‘a’ key four times, then hit the ‘+’ key repeatedly until .5 is displayed. |  |  |  |
|  |  |  |  | The flight must be flown at night and in VFR conditions. If ‘Real weather’ is selected and proves to be unacceptable then the flight must be abandoned. A suitable weather situation may be substituted. It is permissible to commence the flight at dusk to enjoy the early flight scenery and if flying westwards, you may have an opportunity to enjoy the sunsets. |  |  |  |
| KEKO. Elevation – 5,141ft. Runways available. 23 – 233deg. 05 – 053deg. 30 – 298deg. 12 – 118deg. |  | KRNO. Elevation – 4,412ft. Runways available. 34R – 344deg. 16L – 164deg. 34L – 344deg. 16R – 164deg. 25 – 254deg. 07 – 074deg. |  | Morse code ident. 1: · ─ ─ ─ ─ 2: · · ─ ─ ─ 3: · · · ─ ─ 4: · · · · ─ 5: · · · · · 6: ─ · · · · 7: ─ ─ · · · 8: ─ ─ ─ · · 9: ─ ─ ─ ─ · 0: ─ ─ ─ ─ ─ |  |  |  |
| To Waypoint | Waypoint Altitude MSL. (ft) |  | Minimum Enroute Altitude MEA. (ft) | Flight Notes | Hdg (deg) | Dist (nm) |  |
| 0 (zero) 1 (one) 2(two) 3(three) 4(four) | 5,803 7,010 7,977 7,873 9,431 |  | 8,500 10,500 10,500 10,500 12,500 | After take off, climb straight ahead until stabilized, then circle left until able to approach center of airfield at 230deg. Depart on Hdg 230deg and climb to 8,500ft MSL……………………………… Climb to 10,500ft MSL…………………………………………………. Maintain altitude of 10,500ft MSL……………………………………. Maintain altitude of 10,500ft MSL……………………………………. Climb to 12,500ft MSL…………………………………………………. | 230 244 256 233 212 |  | 10 16 23 17 21 |
| To Waypoint | Waypoint Altitude MSL. (ft) |  | Minimum Enroute Altitude MEA. (ft) | Flight Notes | Hdg (deg) | Dist (nm) |  |
| 5(five) 6(six) 7(seven) 8(eight) 9(nine) KRNO | 7,987 3,914 5,334 7,866 6,021 * * * * |  | 10,500 6,500 8,500 10,500 8,500 * * * * | Safe to descend to 10,500ft MSL……………………………………. Safe to descend to 6,500ft MSL……………………………………… Climb to 8,500ft MSL………………………………………………….. Climb to 10,500ft MSL…………………………………………………. Safe to descend to 8,500ft MSL……………………………………… Turn left to 164deg and commence descent to 6,500ft MSL. Make own arrangement for approach and landing. There is a small beacon located on the West side of the airfield roughly a quarter of the way along Rwy 07 which will aid identification of airport. The area around the airport is quite mountainous and maneuvering for landing should take place close to the airport and at 6,500ft MSL until final approach……………………………. | 236 229 224 222 254 164 | 29 22 20 29 14 10 |  |
| Total mileage for flight >>> |  |  |  |  | 211nm |  |  |