Air Mail Night Flight description for FS2004

| Air Mail Night Flight No. AM – RNO – SFO |  |  |  | ALL beacon frequencies are 400.5 KHz. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | In order to hear the Morse identification from the beacon, you must ensure that the ADF sound is turned ‘On’. |  |  |  |
| Reno/Tahoe Intl, Reno, Nevada. (KRNO) To San Francisco Intl, San Francisco, California. (KSFO) |  |  |  | There are two ways to configure the .5 element of the ADF frequency. 1. By setting the frequency in the FSNavigator ADF box. 2. Tune the 400 element in the normal way. Hit the ‘a’ key four times, then hit the ‘+’ key repeatedly until .5 is displayed. |  |  |  |
|  |  |  |  | The flight must be flown at night and in VFR conditions. If ‘Real weather’ is selected and proves to be unacceptable then the flight must be abandoned. A suitable weather situation may be substituted. It is permissible to commence the flight at dusk to enjoy the early flight scenery and if flying westwards, you may have an opportunity to enjoy the sunsets. |  |  |  |
| KSFO. Elevation – 13ft. Runways available. 19L – 192deg. 19R – 192deg. 01L – 012deg. 01R – 012def. 10L – 102deg. 10R – 102deg. 28L – 282deg. 28R – 282deg. |  | KRNO. Elevation – 4,412ft. Runways available. 34R – 344deg. 16L – 164deg. 34L – 344deg. 16R – 164deg. 25 – 254deg. 07 – 074deg. |  | Morse code ident. 1: · ─ ─ ─ ─ 2: · · ─ ─ ─ 3: · · · ─ ─ 4: · · · · ─ 5: · · · · · 6: ─ · · · · 7: ─ ─ · · · 8: ─ ─ ─ · · 9: ─ ─ ─ ─ · 0: ─ ─ ─ ─ ─ |  |  |  |
| To Waypoint | Waypoint Altitude MSL. (ft) |  | Minimum Enroute Altitude MEA. (ft) | Flight Notes | Hdg (deg) | Dist (nm) |  |
| 0 (zero) 1 (one) 2 (two) 3 (three) 4 (four) 5 (five) | 9,010 8.834 4,198 425 80 10 |  | 12,500 12,500 6,500 4,500 4,500 4,500 | After take off, climb straight ahead until stabilized, then circle to approach center of airfield. Turn to 178deg and climb briskly to 12,500ft MSL (700fpm preferred)………………………………… Maintain altitude of 12,500ft. This is a very mountainous area. Safe to Descend to 6,500ftMSL…………………………………….. Safe to descend to 4,500ft MSL. You may remain at 6,500ft MSL to waypoint six if visual contact with the ground permits…….. Maintain altitude of 4,500ft MSL……………………………………. Maintain altitude of 4,500ft MSL……………………………………. | 178 218 215 217 210 217 |  | 22 21 19 25 18 20 |
| To Waypoint | Waypoint Altitude MSL. (ft) |  | Minimum Enroute Altitude MEA. (ft) | Flight Notes | Hdg (deg) | Dist (nm) |  |
| 6 (six) KSFO | 3,530 * * * * |  | 6,500 * * * * | Climb to 6,500ft. Beacon is sited on top of a very high hill…… Commence 500fpm descent to 2,500ft MSL. Do not be misled by Oakland Intl airport which is directly below your route to San Francisco. Determine the runway in use and make own arrangement for landing……………………………………………. | 215 219 | 20 27 |  |
| Total mileage for flight >>> |  |  |  |  | 172nm |  |  |