Air Mail Night Flight description for FS2004

| Air Mail Night Flight No. AM – MLI – IOW |  |  |  | ALL beacon frequencies are 400.5 KHz. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | In order to hear the Morse identification from the beacon, you must ensure that the ADF sound is turned ‘On’. |  |  |  |
| Quad City Intl, Moline, Iowa. (KMLI) To Iowa City Mun, Iowa City, Iowa. (KIOW) |  |  |  | There are two ways to configure the .5 element of the ADF frequency. 1. By setting the frequency in the FSNavigator ADF box. 2. Tune the 400 element in the normal way. Hit the ‘a’ key four times, then hit the ‘+’ key repeatedly until .5 is displayed. |  |  |  |
|  |  |  |  | The flight must be flown at night and in VFR conditions. If ‘Real weather’ is selected and proves to be unacceptable then the flight must be abandoned. A suitable weather situation may be substituted. It is permissible to commence the flight at dusk to enjoy the early flight scenery and if flying westwards, you may have an opportunity to enjoy the sunsets. |  |  |  |
| KMLI. Elevation – 646ft. Runways available. 31 – 306deg. 13 – 126deg. 05 – 047deg. 23 – 227deg. 27 – 270deg. 09 – 089deg. |  | KIOW. Elevation – 669ft. Runways available. 30 – 302deg. 12 – 112deg. 25 – 248deg. 07 – 068deg. 36 – 359deg. 18 – 179deg. |  | Morse code ident. 1: · ─ ─ ─ ─ 2: · · ─ ─ ─ 3: · · · ─ ─ 4: · · · · ─ 5: · · · · · 6: ─ · · · · 7: ─ ─ · · · 8: ─ ─ ─ · · 9: ─ ─ ─ ─ · 0: ─ ─ ─ ─ ─ |  |  |  |
| To Waypoint | Waypoint Altitude MSL. (ft) |  | Minimum Enroute Altitude MEA. (ft) | Flight Notes | Hdg (deg) | Dist (nm) |  |
| 0 (zero) 1 (one) KIOW | 741 717 * * * * |  | 4,500 3,000 * * * * | After take off, climb straight ahead until stabilized, then circle left until able to approach center of airfield at 282deg. Depart on Hdg 282deg and climb to 4,500ft MSL………………………………. It is safe to descend to 3,000ft MSL…………………………………. Turn slight left to 281deg and descend to 2,500ft MSL. Make own arrangement for approach and landing. The beacon located on the East side of the airfield will aid identification of airport. The general area is quite flat with no major obstructions and Maneuvering for landing at 2,500ft MSL is safe…………………… | 282 282 281 |  | 15 18 15 |
| Total mileage for flight >>> |  |  |  |  | 48nm |  |  |