- DC3 Airways – Upstate New York Flight Schedule

Although these flights specify departure and arrival runways, please use the flight description as an illustration of the route only.  As “Pilot-in-command”, your job is to deliver your passengers and cargo to their destination safely and on schedule.  Utilize “Real-World” weather when available and modify your flight plan as required in response to weather conditions &/or VATSIM ATC.  Most of these flights use NDB’s as waypoints, although some rely on visual references such as AIRPORTS. However, feel free to use VOR’s and ILS, but leave the GPS panel behind please!

| Flight Number | Airport From/To | ICAO From/To | Nm/ Time | Flight Description | Altitude |
| --- | --- | --- | --- | --- | --- |
| 3003 | Albany Intl, NY Clinton Co. (Plattsburgh), NY | KALB KPLB | 125 56 | Dep. RWY 1, 012˚to AL NDB 219.0 then 033˚ to GF NDB 209.0 then 027˚ to BT NDB 382.0 then 324˚ to land RWY 32 | 7500 |
| 3004 | Clinton Co., NY Montreal (Ste. Hubert), PQ | KPLB CYHU | 53 26 | Dep. RWY 1, 031˚ to CYJN, then 352˚ to land RWY 36 | 3500 |
| 3005 | Albany Intl, NY Oneida Co. (Utica), NY | KALB KUCA | 76 35 | Dep. RWY 28, 316˚ to JJH NDB 523.0, then 288˚ to CJY NDB 275.0, then 330˚ to land RWY 33 | 6500 |
| 3006 | Albany Intl, NY Oneonta Municipal, NY | KALB N66 | 58 28 | Dep. RWY 28, 273˚ to OZ NDB 279.0, then 237˚ to land RWY 24 | 4500 |
| 3007 | Watertown Intl, NY MacDonald-Cartier (Ottawa), ON | KART CYOW | 84 39 | Dep. RWY 7, 032˚ to 3B NDB 391.0, then 022˚ to YRR NDB 377.0, then 321˚ to land RWY 32 | 5500 |
| 3008 | Oneida Co. (Utica), NY Watertown Intl, NY | KUCA KART | 71 33 | Dep. RWY 33, 033˚ to BK NDB 329.0, then 001˚ to GTB NDB 257.0, then 263˚ to land RWY 25 | 5500 |
| 3009 | Oneonta Municipal, NY Binghamton Regl/Link, NY | N66 KBGM | 52 24 | Dep. RWY 24, 248˚ to BG NDB 332.0, then 340˚ to land RWY 34 | 4500 |
| 3010 | Binghamton Regl/Link, NY Canandaigua, NY | KBGM D38 | 73 33 | Dep. RWY 34, 320˚ to IT NDB 266.0, then 320˚ to SSN NDB 208.0, then 309˚ to land RWY 31 | 6500 |
| 3011 | Canandaigua, NY Greater Rochester Intl, NY | D38 KROC | 21 12 | Dep. RWY 31, 333˚ to RO NDB 400.0, then 278˚ to land RWY 28 | 2500 |
| 3012 | Greater Rochester Intl, NY Buffalo Niagara Intl, NY | KROC KBUF | 49 24 | Dep. RWY 28, 270˚ to BU NDB 231.0, then 234˚ to land RWY 23 | 4500 |
| 3013 | Buffalo Niagara Intl, NY Erie Intl, PA | KBUF KERI | 82 38 | Dep. RWY 23, 233˚ to GB NDB 204.0, then 240˚ to CQD NDB 372.0, then 243˚ to land RWY 24 | 6500 |
| 3014 | Buffalo Niagara Intl, NY Toronto/Pearson Intl, ON | KBUF CYYZ | 72 33 | Dep. RWY 32, 308˚ to SN NDB 408.0, then 359˚ to TZ NDB 257.0, then 345˚ to YZD NDB 356.0, then 251˚ to land RWY 24L | 6500 |
| 3015 | Buffalo Niagara Intl, NY Detroit Metro Wayne Co., MI | KBUF KDTW | 245 145 | Dep. RWY 23, 233˚ to GB NDB 204.0, then 240˚ to CQD NDB 372.0, then 253˚ to LQL NDB 263.0, then 282˚ to PT NDB 283.0, then 307˚ to DT NDB 388.0, then 034˚ to land RWY 3L | 6500 |
| 3016 | Schenectady Co., NY Dutchess Co. (Poughkeepsie), NY | KSCH KPOU | 78 36 | Dep. RWY 22, 180˚ to PFH NDB 272.0, then 195˚ to SKY ACRES K44N, then 247˚ to land RWY 24 | 6500 |
| 3017 | Schenectady Co., NY Bennett (Glens Falls), NY | KSCH KGFL | 33 17 | Dep. RWY 4, 044˚ to GF NDB 209.0, then 012˚ to land RWY 1 | 3500 |
| 3018 | Bennett (Glens Falls), NY Rutland State, VT | KGFL KRUT | 35 18 | Dep. RWY 12, 067˚ to DYO NDB 221.0, then 185˚ to land RWY 19 | 3500 |
| 3019 | Schenectady Co., NY Pittsfield Municipal, MA | KSCH KPSF | 38 19 | Dep. RWY 10, 145˚ to land RWY 14 | 3500 |
| 3020 | Schenectady Co., NY Binghamton Regl/Link, NY | KSCH KBGM | 106 48 | Dep. RWY 28, 265˚ to OZ NDB 279.0, then 246˚ to BG NDB 332.0, then 340˚ to land RWY 34 | 6500 |
| 3021 | Binghamton Regl/Link, NY Wilkes-Barre Scranton Intl., PA | KBGM KAVP | 55 25 | Dep. RWY 16, 160˚ to BG NDB 332.0, then 182˚ to land RWY 16 | 5500 |
| 3022 | Schenectady Co., NY Syracuse Hancock Intl, NY | KSCH KSYR | 98 45 | Dep. RWY 28, 311˚ to JJH NDB 523.0, then 288˚ to CJY NDB 275.0, then 289˚ to SY NDB 242.0, then 280˚ to land RWY 28 | 6500 |
| 3023 | Syracuse Hancock Intl, NY Greater Rochester Intl, NY | KSYR KROC | 69 32 | Dep. RWY 28, 282˚ to RO NDB 400.0, then 278˚ to land RWY 28 | 4500 |
| 3024 | Syracuse Hancock Intl, NY Massena Intl-Richards, NY | KSYR KMSS | 128 57 | Dep. RWY 1, 010˚ to SY NDB 242.0, then 024˚ to GTB NDB 257.0, then 051˚ to MS NDB 278.0, then 046˚ to land RWY 5 | 7500 |
| 3025 | Massena Intl-Richards, NY Adirondack Regional, NY | KMSS KSLK | 48 23 | Dep. RWY 9, 146˚ to SL NDB 395.0, then 228˚ to land RWY 23 | 5500 |
| 3026 | Adirondack Regional, NY Clinton Co. (Plattsburgh), NY | KSLK KPLB | 44 21 | Dep. RWY 5, 048˚ to SL NDB 395.0, then 100˚ to intersect IPLB ILS 111.5 to land RWY 1 | 3500 |
| 3027 | Albany Intl, NY Orange Co. (Montgomery), NY | KALB KMGJ | 85 39 | Dep. RWY 19, 199˚ to PO NDB 403.0, then 267˚ to land RWY 26 | 6500 |
| 3028 | Schenectady Co., NY Bradley Intl (Hartford), CT | KSCH KBDL | 84 39 | Dep. RWY 10, 138˚ to DXT NDB 370.0, then 136˚ to BA NDB 230.0, then 192˚ to land RWY 19 | 5500 |
| 3029 | Schenectady Co., NY Danbury Municipal, CT | KSCH KDXR | 106 48 | Dep. RWY 22, 165˚ to GBR NDB 395.0, then 178˚ to TBY NDB 257.0, then 252˚ to land RWY 26 | 5500 |
| 3030 | Schenectady Co., NY Elmira-Corning Regional, NY | KSCH KELM | 138 61 | Dep. RWY 28, 265˚ to OZ NDB 279.0, then 267˚ to ALP NDB 245.0, then 242˚ to land RWY 24 | 6500 |
| 3031 | Elmira-Corning Regional, NY Buffalo Niagara Intl, NY | KELM KBUF | 114 51 | Dep. RWY 28, 277˚ to EL NDB 269.0, then 309˚ to LYS NDB 360.0, then 339˚ to GB NDB 204.0, then 053˚ to land RWY 5 | 6500 |
| 3032 | Albany Intl, NY Portland Intl, ME | KALB KPWM | 166 73 | Dep. RWY 10, 090˚ to VWD NDB 224.0, then 095˚ to CO NDB 216.0, then 071˚ to PW NDB 394.0, then 112˚ to land RWY 11 | 7500 |
| 3033 | Buffalo Niagara Intl, NY Pittsburgh Intl, PA | KBUF KPIT | 188 82 | Dep. RWY 23, 233˚ to GB NDB 204.0, then 221˚ to ORJ NDB 258.0, then 232˚ to YN NDB 338.0, then 191˚ to EVO NDB 385.0, then 127˚ to land RWY 14 | 6500 |
| 3034 | Buffalo Niagara Intl, NY Cleveland-Hopkins, OH | KBUF KCLE | 168 73 | Dep. RWY 23, 233˚ to GB NDB 204.0, then 240˚ to CQD NDB 372.0, then 253˚ to LQL NDB 263.0, then 241˚ to BKL NDB 416.0, then 236˚ to land RWY 23L | 6500 |
| 3053 | Clinton Co.(Plattsburgh), NY Albany Intl., NY | KPLB KALB | 118 53 | Dep. RWY 19, 196˚ to GF NDB 209.0, then 213˚ to AL NDB 219.0, then 191˚ to land RWY 19 | 6500 |
| 3054 | Montreal (Ste. Hubert), PQ Clinton Co. (Plattsburgh), NY | CYHU KPLB | 50 25 | Dep. RWY 18, 200˚ to land RWY 19 | 4500 |
| 3056 | Oneonta Municipal, NY Albany Intl., NY | N66 KALB | 64 30 | Dep. RWY 6, 082˚ to HEU NDB 356.0, then 125˚ to AL NDB 219.0, then 191˚ to land RWY 19 | 5500 |
| 3057 | MacDonald-Cartier (Ottawa), ON Watertown Intl., NY | CYOW KART | 95 44 | Dep. RWY 17, 173˚ to OG NDB 358.0, then 216˚ to GTB NDB 257.0, then 263˚ to land RWY 25 | 5500 |
| 3058 | Watertown Intl., NY Oneida Co. (Utica), NY | KART KUCA | 71 33 | Dep. RWY 7, 082˚ to GTBNDB 257.0, then 181˚ to BK NDB 329.0, then 150˚ to land RWY 15 | 4500 |
| 3059 | Binghamton Regl/Link, NY Oneonta Municipal, NY | KBGM N66 | 52 25 | Dep. RWY 10, 075˚ to KODEY int using CFB VOR (112.20/257˚) and HNK VOR (116.8/134˚) as bearing points. Proceed 078˚ to RKA VOR 112.60. At RKA, turn right to 133˚ to intercept N66 Rwy 6 ILS 108.90 | 5500 |
| 3060 | Canandaigua, NY Binghamton Regl/Link, NY | D38 KBGM | 82 38 | Dep. RWY 13, 159˚ to ALP NDB 245.0, then 105˚ to land RWY 10 | 5500 |
| 3061 | Greater Rochester Intl., NY Canandiagua, NY | KROC D38 | 22 12 | Dep. RWY 10, 098˚ to RO NDB 400.0, then 154˚ to land RWY 13 | 3500 |
| 3062 | Buffalo Niagara Intl., NY Greater Rochester Intl., NY | KBUF KROC | 51 25 | Dep. RWY 5, 054˚ to BU NDB 231.0, then 099˚ to AVN NDB 344.0, then 042 to land RWY 4 | 3500 |
| 3063 | Erie Intl., PA Buffalo Niagara Intl., NY | KERI KBUF | 82 38 | Dep. RWY 6, 063˚ to CQD NDB 372.0, then 061˚ to GB NDB 204.0, then 053˚ to land RWY 5 | 5500 |
| 3064 | Toronto Pearson Intl, ON Buffalo Niagara Intl., NY | CYYZ KBUF | 65 31 | Dep. RWY 15L, 143˚ to BU NDB 231.0, then 234˚ to land RWY 23 | 5500 |
| 3065 | Detroit Metro Wayne Co., MI Buffalo Niagara Intl., NY | KDTW KBUF | 210 91 | Dep. RWY 9L, 094˚ to DM NDB 223.0, then 090˚ to 4L NDB 236.0, then 085˚ to GB NDB 204.0, then 053˚ to land RWY 5 | 7500 |
| 3066 | Duchess Co. (Poughkeepsie), NY Schenectady Co., NY | KPOU KSCH | 88 41 | Dep. RWY 6, 047˚ to GBR NDB 395.0, then 349˚ to AL NDB, then 305˚ to land RWY 28 | 5500 |
| 3067 | Bennett (Glens Falls), NY Schenectady Co., NY | KGFL KSCH | 32 17 | Dep. RWY 19, 219˚ to land RWY 22 | 2500 |
| 3068 | Rutland State, VT Schenectady Co., NY | KRUT KSCH | 61 29 | Dep. RWY 31, 255˚ to GF NDB 209.0, then 305˚ to land RWY 28 | 4500 |
| 3069 | Pittsfield Municipal, MA Schenectady Co., NY | KPSF KSCH | 39 19 | Dep. RWY 32, 330˚ to AL NDB 219.0, then 305˚ to land RWY 28 | 4500 |
| 3070 | Binghamton Regl/Link, NY Schenectady Co., NY | KBGM KSCH | 99 45 | Dep. RWY 10, 076˚ to OZ NDB 279.0, then 085˚ to land RWY 10 | 5500 |
| 3071 | Wilkes-Barre Scranton Intl., PA Binghamton Regl/Link, NY | KAVP KBGM | 54 26 | Dep. RWY 34, 003˚ to BG NDB 332.0, then 340˚ to land RWY 34 | 5500 |
| 3072 | Syracuse Hancock Intl., NY Schenectady Co., NY | KSYR KSCH | 99 45 | Dep. RWY 10, 108˚ to CJY NDB 275.0, then 108˚ to JJH NDB 523.0, then 131˚ to land RWY 10 | 7500 |
| 3073 | Greater Rochester Intl., NY Syracuse Hancock Intl., NY | KROC KSYR | 68 32 | Dep. RWY 10, 098˚ to RO NDB 400.0, navigate using SY NDB 242.0 course 103˚ to land RWY 10 | 5500 |
| 3074 | Massena Intl.-Richards, NY Syracuse Hancock Intl., NY | KMSS KSYR | 130 59 | Dep. RWY 27, 251˚ to OG NDB 358.0, then 216˚ to GTB NDB 257.0, then 204˚ to SY NDB 242.0, then 280˚ to land RWY 28 | 6500 |
| 3075 | Adirondack Regional, NY Massena Intl.-Richards, NY | KSLK KMSS | 53 25 | Dep. RWY 5, 048˚ to SL NDB 395.0, then 318˚ to MS NDB 278.0, then 046˚ to land RWY 5 | 5500 |
| 3076 | Clinton Co.(Plattsburgh), NY Adirondack Regional, NY | KPLB KSLK | 35 18 | Dep. RWY 32, 259˚ to SL NDB 395.0, then 228˚ to land RWY 23 | 4500 |
| 3077 | Orange Co. (Montgomery), NY Albany Intl., NY | KMGJ KALB | 81 38 | Dep. RWY 3, 042˚ to PFH NDB 272.0, then 007˚ navigating using AL NDB 219.0 to land RWY 1 | 5500 |
| 3078 | Bradley Intl.(Hartford), CT Schenectady Co. NY | KBDL KSCH | 80 37 | Dep. RWY 33, 340˚ to DXT NDB 370.0, then 321˚ to AL NDB 219.0, then 305˚ to land RWY 28 | 4500 |
| 3079 | Danbury Municipal, CT Schenectady Co., NY | KDXR KSCH | 94 43 | Dep. RWY 35, 002˚ to PFH NDB 272.0, then 008˚ to AL NDB 219.0, then 305˚ to land RWY 28 | 5500 |
| 3080 | Elmira-Corning Regional, NY Schenectady Co., NY | KELM KSCH | 140 62 | Dep. RWY 10, 067˚ to IT NDB 266.0, then 094˚ to OZ NDB 279.0, then 088˚ navigate to AL NDB until intercepting ILS 109.7 to land RWY 4 | 7500 |