DC3 Airways - Venezuela Flights Schedule

By Daniel Nole

DCA-116

A set of national and International flights of DC-3 Airways,in the wondefull land of Venezuela and Caribbean Area.

Although these flights specify departure and arrival runways, please use the flight description as an illustration of the route only.  As “Pilot-in-command”, your job is to deliver your passengers and cargo to their destination safely and on schedule.  Utilize “Real-World” weather when available and modify your flight plan as required in response to weather conditions &/or VATSIM ATC.Actual navaids and routes were utilised for build of this flight plans.However, feel free to use,NDB's VOR’s and ILS, but leave the GPS panel behind and utilise not too much FSNav !

NOTE:Actually Rwy 09/27 and 08/26 were changed in SVMI Maiquetía Intenational from two years; 09/27 is now 10/28 and 08/26 is 09-27 but for reasons of standarisation

I have decided to utilise de default old Rwy numbers included in FS2004.I hope Microsoft give with his next future relase of FS 2006(or FS10) the update of this Rwys.

| Flight Number | Airport From/To | ICAO From/To | Nm/ Time | Flight Description | Altitude |
| --- | --- | --- | --- | --- | --- |
| VE-001 | Maiquetia Simon Bolivar Intl Barcelona,Gen. Jose Antonio Anzoategui | SVMI SVBC | 154 66 | Set VOR to MIQ 114.80 Dep. RWY 08, 082° for 3.5 nm, then 95° for 13nm.With 16nm from VOR MIQ turn to 104° and change NAV1 to VOR CBC 113.50. Over VOR CBC change freq. to VOR BNA 115.90 and turn to 119°to follow Airways G443.Start descend to 3200 ft, 21 nm from VOR 115.90.Checking VOR 115.90 and starting down-wind HDG 210° for 6.5 nm,then base 148°(1.5 nm) ,88°(1.5 nm) and 19° to final and landing SVBC Rwy 02(ALT 26 ft) | 9000 |
| VE-002 | Barcelona,Gen. Jose Antonio Anzoategui Margarita,Del Caribe Intl Gen. Santiago Mariño | SVBC SVMG | 73 36 | Set VOR to BNA 115.90 Dep. RWY 02, 018˚for 6.5nm, then turn to 76° and change VOR to CUM 113.30.Checking VOR 113.30 and turn to 22°.Set ADF to NDB MT 218.0.Start descend to 950 ft, 10.5 nm after VOR CUM.Over MT turn to 90° for final and landing SVMG Rwy 09(ALT 75 ft).ILS 110.10 Available. | 5000 |
| VE-003 | Margarita,Del Caribe Intl Gen. Santiago Mariño Maiquetia Simon Bolivar Intl | SVMG SVMI | 212 91 | Set VOR to MTA 114.10 Dep. Rwy 09, 89˚ for 5 nm, then 140˚(1.5nm)then 191°(2 nm),and then 246˚(8.5 nm).After 273° to follow Airways A561 and change VOR to CBC 113.50.Over VOR 113.50 turn to 296° HDG INT OSMAR.Over INT OSMAR and with 31.5nm from VOR CBC turn to 270° and change NAV1 to VOR MIQ 114.80.Start to descend to 2150 ft, 22nm from VOR MIQ.With 2150 ft ALT and 7.7nm from VOR MIQ turn base with 189° and continued descend to 1500 ft.Finally with 7.3nm from VOR MIQ turn 96° for final,capture LOC and landing SVMI Rwy 09(ALT 236 ft).ILS 110.10 available. | 10000 |
| VE-004 | Maiquetia Simon Bolivar Intl Calabozo | SVMI SVCL | 146 68 | Set VOR to MIQ 114.80 Dep. RWY 08, 082° for 4.5 nm, then 76° for 13.2nm HDG INT D081Q.Climb to gain altitude.Over INT D081Q and with 16.8nm from VOR MIQ turn to 193° HDG INT ESANA and continued climbing.Over INT ESANA and with 16.4nm from VOR MIQ turn to 206° and change NAV1 to VOR TUY 115.20. Over VOR TUY turn to 235° to follow Airways W17A and set ADF to NDB SSB 318.0 Checking NDB 318.0 and turn to 203°to follow now Airways W17.Change NAV1 to VOR CZO 116.30.With 20.8nm from VOR CZO start descend to 3000 ft.With 2.8nm from VOR CZO turn to 270° for start lefthand traffic pattern downwind and continued descend to 1800.Turn base with 177°,5.4nm from VOR CZO and with 4.7nm turn to 89° for final and landing SVCL Rwy 09(ALT 328 ft). | 8000 |
| VE-005 | Calabozo San Fernando De Apure | SVCL SVSR | 78 39 | Dep. Rwy 09, 090˚(3 nm)then 130° (2.5nm)and 210° for 12.3nm to take Airway W17.Set NAV1 to VOR SFD 112.10.With 49.4nm from VOR SFD turn to 190° to entry Airways W17. With 14.6nm from VOR SFD start descend to 3100 ft.With 3.4 nm from VOR SFD turn to 290°for start lefthand traffic pattern downwind and continued descend to 1900 ft.With 6.5nm from VOR SFD turn base with 207° for 3nm and then 115° for final and landing SVSR Rwy 11(ALT 157 ft). | 6000 |
| VE-006 | San Fernando De Apure Puerto Ayacucho | SVSR SVPA | 153 67 | Dep. Rwy 11, 111˚ for 3nm,then 153°(3 nm)and then 226°.Set NAV1 to VOR SFD 112.10 and ADF to PCR 353.0.With 8.3nm from VOR SFD turn to 190° for follow Airways W17.Change then NAV1 to VOR PAY 112.80.Over NDB PCR turn to 199°With 17.9nm from VOR PAY start descend to 3600 ft .With 2.8nm from VOR PAY turn to 277° for 2.5nm and then 205° to start lefthand downwind pattern traffic and continued descend to 1500 ft.Don't cross the Colombian border.With 4.8nm from VOR PAY turn base with 116° and finally turn 26° for final and landing SVPA Rwy 03(ALT 246 ft). | 8000 |
| VE-007 | Puerto Ayacucho Maiquetia Simon Bolivar Intl | SVPA SVMI | 350 142 | Plane catering.Night flight ,departing at 18.20.Set NAV1 to VOR PAY 112.80. Dep. Rwy 21, 210˚ for 3nm,then 135° for 3nm,80° for again 3nm,then 8° to take Airways W13F.With 7.1nm from VOR PAY turn to 49° and set ADF to NDB PIJ 310.0.Over NDB PIJ and with 74.6nm from VOR PAY turn to 343° and change NAV1 to VOR SFD 112.10.Over VOR SFD turn to 10° to follow Airways W17 and change NAV1 to VOR CZO 116.30.Checking VOR CZO and turn to 23° HDG INT ALTOS.Change again NAV1 to VOR NOL 116.10.With 13nm from VOR NOL start slow descend to 6100 ft.Over INT ALTOS and with 7.4nm from VOR NOL turn to 336° HDG INT miq11 and continued slow descend to 2400 ft.Keep in eye the mountains peaks or your Radioltimeter(if you have one).Dont descend bellow 3300 ft before the coast line or between 7.4nm from VOR NOL.Over INT miq11 and with 9.5nm from VOR NOL turn to 96° for capture LOC, final and landing SVMI Rwy 09(ALT 236 ft). ILS 110.10 available | 9000 |
| VE-008 | Maiquetia Simon Bolivar Intl Puerto Cabello Gen.B.Salom | SVMI SVPC | 77 38 | Dep. Rwy 08, 82˚ for 3.5nm,then 38° for 1.5nm,9° for 1.5nm and 316° also for 1.5nm.Then 219° HDG INT TORIM.Set NAV1 to VOR PBL 117.70.Over INT TORIM and with 32nm from VOR PBL turn to 256° to follow Airways W5 to INT pb11a.With 25nm from VOR PBL start descend to 2400 ft.Over INT pb11a and with 10.8nm from PBL turn to 234° and continued descend to 1100 ft.With 6.1nm from VOR PBL turn to 280° for final and landing SVPC Rwy 28(ALT 33 ft). | 6000 |
| VE-009 | Puerto Cabello Gral.B.Salom Coro Jose L.Chirinos Intl | SVPC SVCR | 124 61 | Dep. Rwy 28, 280˚for 3.5nm, then 308˚HDG INT 1169N.Set NAV1 to VOR CRO 113.30.Over INT 1169N and with 47.7nm from VOR CRO turn to 329° HDG INT COZAL.Over INT COZAL and with 25nm from VOR CRO turn to 288° to follow Airways W1.Start to descend to 3200 ft, 22.4nm from VOR CRO.With 5nm from VOR CRO turn to 265° to start righthand traffic pattern and continued descend to 1300 ft.With 4.2nm from VOR CRO turn base with 348°for 2nm and then 85° for final and landing SVCR Rwy 08(ALT 52 ft). | 8000 |
| VE-010 | Coro Jose L.Chirinos Intl Maracaibo La Chinita Intl | SVCR SVMC | 150 67 | Dep. Rwy 26, 266˚for 4nm,then 247° HDG INT MAMON and to follow Airways G341,set NAV1 to VOR MAU 117.90.Over INT MAMON and with 62.4nm from VOR MAU small correction to 248° to entry Airways G341.Over VOR MAU turn to 267 to follow Airways A552.Change NAV1 to VOR MAR 115.70.With 12.4nm from VOR MAR start descend to 5200 ft.With 2.8nm from VOR MAR turn to 201° to start righthand traffic pattern and continued descend to 2500 ft.With 9.8nm from VOR MAR turn base with 293° for 3.5nm.Then 26° for capture LOC,final and landing SVMC Rwy 02L(ALT 236 ft).ILS 110.10 available | 8000 |
| VE-011 | Maracaibo La Chinita Intl Valera Dr Antonio N Briceño | SVMC SVVL | 108 51 | Set NAV1 to VOR MAR 115.70. Dep. Rwy 02L, 26˚for 3nm, then 74° for 2nm and after 153° HDG INT d145m.Over INT d145m and with 13nm from VOR MAR turn to 145° to follow Airways W7 and HDG INT TEGER.Over INT TEGER and with 34.5nm from VOR MAR small correction to 144° HDG INT ERIXO.Change NAV1 to VOR VRA 114.40.With 14.2nm from VOR VRA start slow descend to 5700 ft.Over INT ERIXO and with 10.3nm from VOR VRA turn to 115° and continued descend to 3100 ft.Finally over VOR VRA turn to 203° for final and landing SVVL Rwy 20(ALT 2060 ft). | 7000 |
| VE-012 | Valera Dr Antonio N Briceño Merida Alberto Carnevalli | SVVL SVMD | 59 32 | Set NAV1 to VOR VRA 114.40 Dep. RWY 20, 198˚ for 3nm ,then 221° for 10nm.With 17.4nm from VOR VRA and minimum ALT 6100 ft,turn to 203° for 10nm.With 27.2nm from VOR VRA and minimum ALT 8900 ft,turn to 266° for 12nm.With 35nm from VOR VRA and minimum ALT 11.400 ft turn to 165° for 12nm.With 42.4nm from VOR VRA and with minimum ALT 13.700 ft.turn to 282°.With 45.3nm from VOR VRA start very slow descend to 12.100 ft.Keep in eye the mountains peaks or your radioaltimeter(If you have one).With 49.1nm from VOR VRA and minimum ALT 12.000 ft turn to 251° and continued descend to 10.100 ft.With 54.8nm from VOR VRA and minimum ALT 10.000 ft turn to 239° and continued descend to 6700 ft.With 65.6nm from VOR VRA turn base with 149° and continued descend to 6100 ft.Finally,with 66.1nm from VOR VRA turn to 62° for final and landing SVMD Rwy 06(ALT 5007 ft). Oxigen supply required,prop deice and fast climb recomended(800-900 fpm) and ev.orbiting over SVVL to gain ALT to climb the mountains.Reduced fuel load also recomended.(350-400 Gals) | 14000 |
| VE-013 | Merida Alberto Carnevalli San Antonio del Táchira | SVMD SVSA | 151 61 | Set your mix parameters in relation with your actual ALT:5006 ft Dep. RWY 06,HDG 62° for 25nm, gain ALT,then turn to 96° for 1.5 nm and 6° for another 1.5nm and finally 276° again for 1.5nm.Then HDG 231°for 5.5nm and after 243° for 21nm.Set NAV1 to VOR LFA 113.00.At 68nm from 113.00 turn to 258° for 22nm.48nm from VOR 113.00 start first descend to 12.500 ft.With 30nm from VOR 113.00 a second descend phase to 11.500 ft.Third descend to 8400 ft with 11nm from VOR 113.00. Over LFA VOR 113.00 turn to 223° for 20nm and start again descend to 2800 ft(-700fpm).Set NAV1 to VOR CUC 113.30.At 4nm from VOR CUC turn to 202° for 2.0nm and continued descend to 2300 ft.Finally with 2nm from radial VOR CUC turn to 160° for final in SVSA Rwy 16(ALT 1312 ft). .Oxigen supply required,prop deice and fast climb recomended(800-900 fpm).Reduced fuel load also recomended.(250-300 Gals) | 14000 |
| VE-014 | San Antonio del Táchira Barinas | SVSA SVBI | 165 75 | Dep. Rwy 16,HDG 161° for 6nm,then 89°.Set ADF to NDB PRM 310.Over NDB PRM turn to 117°and follow Airways G300.Set NAV1 to VOR CTN 116.00.Checking VOR CTN and turn to 55° to follow Airways A550 HDG INT ERICA.Change NAV1 to VOR BNS 117.50.With 32nm from VOR BNS start descend to 3000 ft.Over INT ERICA and with 10 nm from VOR BNS turn to 71° for base and continued descend to 1500 ft.With 4.5nm from VOR BNS turn to 28° for landing SVBI Rwy 03(ALT 666 ft). | 9000 |
| VE-015 | Barinas Barquisimeto | SVBI SVBM | 103 49 | Dep Rwy 03, 30° for 3nm,then turn to 41° HDG INT CHAPA.Set VOR to BNS 117.50.Over INT CHAPA and with 9.8nm from VOR BNS turn to 37° to follow Airways W22 and HDG INT ASUPO.Over INT ASUPO and with 60nm from VOR BNS small correction to 38° HDG INT MIMET.Change NAV1 to VOR BRM 112.20.Over INT MIMET and 24.2nm from VOR BRM rest in HDG 38°.Start to descend to 4200 ft, 23 nm from VOR BRM.With 6.4nm from VOR BRM turn to 347° and set ADF to NDB BR 363.0.Continued descend to 3000 ft. Over NDB BR turn to 88° for final and landing SVBM Rwy 09(ALT 2041 ft). | 9000 |
| VE-016 | Barquisimeto Maiquetia Simon Bolivar Intl | SVBM SVMI | 164 76 | Night flight, departing 19.15.Set NAV1 to VOR BRM 12.20 and ADF to NDB BR 363.0. Dep.Rwy 27,268° HDG NDB BR .Over NDB BR turn to 308° for 2nm, then 8° for others 2nm and last 52° for again 2nm.With 6.5nm from VOR BRM turn to 107° and change ADF to NDB YAR 260.0.Over NDB YAR turn to 74° HDG INT KERON and follow Airways W5.Change NAV1 to VOR PBL 117.70.Maintain same HDG of 74° after checking INT KERON with 40nm from VOR PBL.Over VOR PBL turn to 77° HDG INT TORIM and continued over Airways W5.Over INT TORIM and with 34nm from VOR PBL turn to 113° HDG INT miq14 and change NAV1 to VOR MIQ 114.80.Start to descend to 3000 ft,28.5nm from VOR MIQ.With 14 nm from VOR MIQ turn to 96° for capture LOC,final and landing SVMI Rwy 09(ALT 236 ft). | 7000 |
| VE-017 | Maiquetia Simon Bolivar Intl Cumana MariscaL Antonio Jose de Sucre | SVMI SVCU | 170 79 | Dep. Rwy 08, 82˚ for 8nm,then turn to 98°.Set NAV1 to VOR CBC 113.50. With 25.8nm from VOR CBC turn to 110°.Checking VOR CBC and turn to 104° HDG INT MEXOT and follow Airways A552.Over INT MEXOT change NAV1 to VOR CUM 113.30 and continued with same HDG of 104° to INT PORGA.With 28nm from VOR CUM start descend to 1300 ft.Over INT PORGA and with 24.3nm from VOR CUM turn to 112° and continued descend.With 6.3nm from VOR CUM and over the coast line turn to 78° for final and landing SVCU Rwy 08(ALT 13 ft). | 7000 |
| VE-018 | Cumana MariscaL Antonio Jose de Sucre Maturin Intl. | SVCU SVMT | 80 40 | Dep.Rwy 08,79° for 6nm,then 201° for 2nm and after 148°.Set ADF to NDB MUN 230.0 and VOR to MUN 115.30.Start descend to 1700 ft 24nm from VOR MUN.With 7nm from VOR MUN turn to base with 101° for 2nm and then 61° for landing SVMT Rwy 05(ALT 223 ft). | 7000 |
| VE-019 | Maturin Intl. Ciudad Guayana, Manuel Carlos Piar | SVMT SVPR | 107 52 | Set NAV1 to VOR MUN 115.30. Dep.Rwy 05, 60° for 4nm,then 101° for others 2nm and then 202°.With 9.2nm from VOR MUN turn to 176 to follow Airways A300.Change then NAV1 to VOR GNA 114.70.With 18nm from VOR GNA start descend to 3100 ft.With 3nm from VOR GNA turn to 248° to start lefthand pattern traffic downwind and continued descend to 2000 ft.With 5nm from VOR GNA turn base with 156° for 3nm and last, turn 68° for final and landing SVPR Rwy 07(ALT 469 ft). | 7000 |
| VE-020 | Ciudad Guayana, Manuel Carlos Piar Ciudad Bolivar | SVPR SVCB | 57 29 | Set NAV to VOR GNA 114.70. Dep.Rwy 25, 250° for 4nm,then 284°.With 6nm from VOR GNA turn to 268° to follow Airways W7.Then change NAV1 to VOR CBL 115.10.With 16.5nm from VOR CBL start descend to 1900 ft.Over VOR CBL small correction to 270° and 2 nm after VOR CBL turn to 294° and continued descend to 1100 ft.With 3.5nm from VOR CBL turn base with 11° for 1.5nm and then 116° for final and landing SVCB Rwy 12(ALT 197 ft). | 6000 |
| VE-021 | Ciudad Bolivar Barcelona,Gen. Jose Antonio Anzoategui | SVCB SVBC | 153 71 | Set NAV1 to VOR CBL 115.10. Night flight service.Dep.Rwy 30, 298° for 4nm ,then 321 for 3nm, after 349°.With 15.5nm from VOR CBL turn to 355° to follow Airways G678.Set ADF to NDB SOM 220.0. Over NDB SOM turn to 345° to follow now Airways W16 and change NAV1 to VOR BNA 115.90.Start descend to 4000 ft ,21nm from VOR BNA.With 5.5nm from VOR BNA turn to 357° to start lefthand pattern traffic downwind and continued descend to 1800 ft.With 4.5nm from VOR BNA turn base with 256° for 3nm and then turn to 152° for final and landing SVBC Rwy 15(ALT 26 ft) | 8000 |
| VE-022 | Barcelona,Gen. Jose Antonio Anzoategui Maiquetia Simon Bolivar Intl | SVBC SVMI | 167 78 | Set NAV1 to VOR BNA 115.90. Plane catering.Night last flight service.Dep.Rwy 20, 198° for 3nm,then 251° for 2nm and 281° for anothers 2nm.Then 335° for 9nm. With 8.6nm from VOR BNA turning to 299° to follow Airways G443 and HDG INT ISULA.Over INT ISULA and with 40nm from VOR BNA small correction to 298° and change NAV1 to VOR CBC 113.50.Over VOR CBC turn to 296° HDG INT OSMAR.Over INT OSMAR and with 31.5nm from VOR CBC turn to 273° and change NAV1 to VOR MIQ 114.80.Start descend to 2700 ft, 12.5nm from VOR MIQ.With 9.2nm from VOR MIQ turn base with 185° and continued descend to 1700 ft.Finally, with 8.4nm from VOR MIQ, turn to 96 for capture LOC, final and landing SVMI Rwy 09(ALT 236 ft).ILS 110.10 available. | 8000 |
| VE-023 | Ciudad Bolivar Canaima | SVCB SVCN | 140 67 | Set NAV1 to VOR CBL 115.10 Dep.Rwy 24, 239° for 3.5nm, then turn to 190° for 1.5nm and after 130°for 5nm.With 5.8nm from VOR CBL turn to 171° and follow Airways W20F.Then change NAV1 to VOR CMA 117.50.With 8.2nm from VOR CMA start descend to 4900 ft. With 6.4nm from CMA turn to 199°to start lefthand pattern traffic downwind and continued descend to 3100 ft.With 6.3nm from VOR CMA turn base with 93° and continued descend to 2500 ft and last, with 5.9nm turn to 357° for final and landing SVCN Rwy 36(ALT 1450 ft). | 7000 |
| VE-024 | Canaima Ciudad Bolivar | SVCN SVCB | 132 62 | Set NAV1 to VOR CMA 117.50 Dep.Rwy 36,360° for 3nm,then 346°.With 6nm from VOR CMA turn to 352° to follow Airways W20F.Change after NAV1 to VOR CBL 115.10.Start descend to 4000 ft ,15 nm from VOR CBL. With 7.5 nm from VOR CBL, turn downwind with 24° and continued descend to 1400 ft.With 6 nm from VOR CBL turn base with 181° and continued descend to 1000 ft.Then,turn to 239° for final and landing SVCB Rwy 24(ALT 197 ft). | 6000 |
| VE-025 | Maiquetia Simon Bolivar Intl Los Roques | SVMI SVRS | 88 46 | Set NAV1 to VOR MIQ 114.80. Early morning flight.Dep. Rwy 08, 82° for 4nm,then turn to 11° HDG INT DARPA.Over INT DARPA and with 17nm from VOR MIQ turn to 21° to follow Airways G432 and HDG INT Mi54B.Change then NAV1 to VOR LRS 113.10.Over INT Mi54B and with 28 nm from VOR LRS small HDG correction to 24°.With 19nm from VOR LRS start descend to 1400 ft.With 4nm from VOR LRS turn to base with 334° continuing descend to 700 ft, and then 80° for final and landing SVRS Rwy 07(ALT 1 ft). | 5000 |
| VE-026 | Los Roques Maiquetia Simon Bolivar Intl | SVRS SVMI | 101 50 | Set NAV1 to VOR LRS 113.10. Dep. Rwy 07, 79° for 2.5 nm,after turn to 158° for 2 nm and then 226°. With 8.3nm from VOR LRS turn to 203° to follow Airways G432 HDG INT Mi54B.Over INT Mi54B and with 28 nm from VOR LRS small HDG correction to 201°HDG INT DARPA.Change NAV1 to VOR MIQ 114.80.Over INT DARPA and with 17nm from MIQ another small HDG correction to 202°.Start to descend to 4100 ft, 10.5nm from VOR MIQ.With 3.7nm from VOR MIQ turn downwind with 270° and continued descend to 2200 ft.With 7.7nm from VOR MIQ turn base with 189° for 3nm and then 96° for capture LOC,final and landing SVMI Rwy 09 (ALT 236 ft).ILS 110.10 available | 6000 |
| VE-027 | Barcelona,Gen. Jose Antonio Anzoategui Maturin Intl | SVBC SVMT | 99 51 | Set NAV1 to VOR BNA 115.90. Dep.Rwy 15,153° for 3nm,after turn to 89°.With 8.6 nm from VOR BNA turn to 115° HDG INT PARDU and follow Airways G443.Over INT PARDU and with 40nm from VOR BNA small correction to 116° and change NAV1 to VOR MUN 115.30. Start descend to 2800 ft, 24nm from VOR MUN.With 7.8 nm from VOR MUN turn base with 159° and continued descend to 1300 ft and last with 5.4nm turn to 58° for landing SVMT Rwy 05(ALT 223 ft). | 7000 |
| VE-028 | Maturin Intl Barcelona,Gen. Jose Antonio Anzoategui | SVMT SVBC | 107 49 | Set NAV1 to VOR MUN 115.30. Dep. Rwy 05,60° for 3nm,then 344° for 2.5nm and after 281°.With 16nm from VOR MUN turn to 295° HDG INT PARDU and follow Airways G443.Over INT PARDU with 54.8 nm from VOR MUN, small correction to 294° and change NAV1 to VOR BNA 115.90.Start descend to 2500 ft, 26nm from VOR BNA.With 5.5nm from VOR BNA turn to 316° to start lefthand downwind traffic pattern and continued descend to 1000 ft. With 2.8nm from VOR BNA turn base with 231° and then 153° for final and landing SVBC Rwy 15(ALT 26 ft). | 8000 |
| VE-029 | Maturin Intl Tucupita | SVMT SVTC | 82 40 | Set NAV1 to VOR MUN 115.30. Dep.Rwy 05,60° for 4 nm,then turn to 102° for 2.5 nm and after 176°. With 8nm from VOR MUN turn to 134° to follow Airways G443. Set ADF to to NDB TUC 225.0 With 55nm from VOR MUN start slow descend to 600 ft.Over NDB TUC with 600 ft ALT and airfield in sight turn to 72° for final in SVTC Rwy 07(ALT 33 ft).Caution:No fuel available in this airfield. | 5000 |
| VE-030 | Tucupita Maturin Intl | SVTC SVMT | 94 47 | Set NAV1 to VOR MUN 115.30. Dep.Rwy 07,71° for 4nm,then turn to 12° for 2 nm and after 282°.With 65.8nm from VOR MUN turn to 314° to follow Airways G443.Start to descend to 2800 ft 15nm from VOR MUN.With 2.8nm from VOR MUN turn downwind with 236° and continued descend to 1800 ft.With 5nm from VOR MUN turn base with 326° for 3nm and then 58° for final and landing SVMT Rwy 05(ALT 223 ft). | 6000 |
| VE-031 | Maiquetia Simon Bolivar Intl Guanare | SVMI SVGU | 220 97 | Dep.Rwy 08,82° for 4 nm,then 34° for 2nm and 343° for another 2nm.Then MARES 08 Depart procedure,turn to 301° for 17nm.Turn NAV1 to VOR 117.70 PBL Over INT MARES turn to 259° to VOR PBL.Checking VOR PBL and turn to 238° to follow Airways A550.Set ADF to NDB 225.0 AGV and NAV1 to VOR 113.40 AGV.Over VOR AGV turn to 231° and change ADF to NDB 318.0 GRE.Start to descend to 2400 ft 19.7nm from VOR AGV. Start Lefthand pattern over NDB GRE ,233° for 3.5 nm, then base with 140° and final in the last 3nm with 48° for landing SVGU Rwy 05(ALT 607 ft). | 9000 |
| VE-032 | Guanare Santo Domingo-Mayor Buenaventura Vivas Intl | SVGU SVSO | 183 80 | Dep.Rwy 05,48° for 3 nm, then turn to 351° for 1.5nm, after 303° for again 1.5nm and finally 232° to follow Airways A552.Set NAV1 to VOR BNS 117.50.Over VOR BNS turn to 235° and set ADF to NDB SBB 336.0.Over NDB SBB, turn to 264° and turn NAV1 to DME STD 115.30. Start to descend to 2500 ft, 20.5nm from DME STD.With 6 nm from DME STD and in engaged in lefthand pattern traffic turn to 185° for base and after 2.5nm turn to 112° for final in SVSO Rwy 11(ALT 1082ft).Caution with militar traffic in the area. | 9000 |
| VE-033 | Santo Domingo-Mayor Buenaventura Vivas Intl Guanare | SVSO SVGU | 166 73 | Dep.Rwy 11,109° for 2.5nm then turn to 79°.Set ADF to NDB SBB 336.0.Over NDB SBB turn to 56°to follow Airways A550 and set NAV1 to VOR BNS 117.50 .Start to descend to 1500 ft, 1.5nm after VOR BNS and turn to 58°.With 32nm from VOR BNS and with the airfield in sight turn to 47° for final in SVGU Rwy 05(ALT 607 ft). | 10000 |
| VE-034 | Guanare Maiquetia Simon Bolivar Intl | SVGU SVMI | 196 88 | Flight departing 18.30.Dep Rwy 05,49° for 4nm.Then turn to 50° to follow Airways A550 and set ADF to NDB AGV 225.0.Over NDB AGV turn to 59° and set NAV1 to VOR PBL 117.70.Over VOR PBL turn to 92° and start to descend to 3000 ft ,32.5 nm after VOR PBL.Then change NAV1 to ILS IMIQ 110.10.Over INT miq14 and 12 nm from destination turn to 96° to get the LOC and final in SVMI Rwy 09(ALT 236 ft). | 8000 |
| VE-035 | Maiquetia Simon Bolivar Intl Valencia Arturo Michelena Intl. | SVMI SVVA | 107 52 | Set NAV1 to VOR MIQ 114.80. Dep.Rwy 08,82° for 3nm,then turn to 44° for 1.5nm and after 10° for anothers 1.5nm.Then 104° for a MARES 08 Departure.Over INT Mares and with 17nm from VOR MIQ turn to 259° to follow Airways W5 and change NAV1 to VOR PBL 117.70.Over VOR PBL turn to 166° and follow Airways W15.With 3.4nm from VOR PBL start slow descend to 5000 ft and change NAV1 to VOR VAL 117.40.With 8.6nm from VOR VAL, turn base with 218° and continued descend to 3100 ft.Finally with 8nm from VOR VAL turn to 102° for final and landing SVVA Rwy 10(ALT 1421 ft). | 8000 |
| VE-036 | Valencia Arturo Michelena Intl. Maiquetia Simon Bolivar Intl | SVVA SVMI | 65 31 | Set NAV1 to VOR VAL 117.40. Dep.Rwy 10,101° for 4nm.Over de Valencia Lake and with 3nm from VOR VAL turn to 55° for 27nm.Avoid Militar exercises area and keep in eye the mountains.With 29nm from VOR VAL turn to 71° for 18nm HDG INT D287Q.Start slow descend to 3800 ft with 35 nm from VOR VAL.Over INT D287Q and with 46.7nm from VOR VAL,turn to 102° for 3nm HDG INT miq14. Change NAV1 to ILS IMIQ 110.10.Over INT miq14 and 12 nm from destination turn to 96° to get the LOC and final in SVMI Rwy 09(ALT 236 ft). | 7000 |
| VE-037 | Maracaibo La Chinita Intl Paraguana Josefa Camejo Intl | SVMC SVJC | 120 58 | Dep.Rwy 02L,26° for 4nm.Then turn to 57° HDG INT GOLFO and follow Airways W1.Set NAV1 to VOR MAR 115.70.Over INT GOLFO and with 40nm from VOR MAR turn to 60° HDG INT CAUCA.Change NAV1 to VOR PRG 113.60.Over INT CAUCA and with 31.8nm from VOR PRG turn to 54°.With 29nm from VOR PRG start descend to 1200 ft.And with 6.7nm from VOR PRG turn to 88° for final in SVJC Rwy 09(ALT 75 ft). | 7000 |
| VE-038 | Paraguana Josefa Camejo Intl Coro Jose L.Chirinos Intl | SVJC SVCR | 39 19 | Set NAV1 to VOR PRG 113.60. Dep.Rwy 09,88° for 3.5nm,then turn to 120° for 1.5nm and after 173°. With 5nm from VOR PRG turn to 138° to follow Airways W1. Then change NAV1 to VOR CRO 117.30.With 19 nm from VOR CRO start descend to 2200 ft.With 7.6 nm from VOR CRO turn base with 169° and continued descend to 1000 ft. And with 4nm from VOR CRO turn to 86° for final and landing SVCR Rwy 08(ALT 52 ft). | 5000 |
| VE-039 | Coro Jose L.Chirinos Intl Paraguana Josefa Camejo Intl | SVCR SVJC | 56 28 | Set NAV1 to VOR CRO 117.30. Dep.Rwy 08,86° for 4nm,then 39° for 1.8nm and 332° for another 1.8nm.Then to 277°.With 6nm from VOR CRO turn to 318° to follow Airways W1.Change NAV1 to VOR PRG 113.60. Start to descend to 2700 ft 5.6 nm from VOR PRG.Over VOR PRG and in cross wind, start lefthand pattern.2.4nm after VOR PRG turn to 266° for downwind for 4nm.Then base with 175° for 1.5nm and finally turn 88° for final with 4nm in SVJC Rwy 09 (ALT 75 ft). | 4000 |
| VE-040 | Paraguana Josefa Camejo Intl Maracaibo La Chinita Intl | SVJC SVMC | 149 69 | Set NAV1 to VOR PRG 113.60. Dep.Rwy 09,88° for 3.5nm, then 34° for 1.5nm and 346° for another 1.5nm.After turn to 269° for 9.5nm. Over the coast line and with 7.7nm from VOR PRG turn to 212° HDG INT PUNTA.Over INT PUNTA and with 30nm from VOR PRG turn to 225° to follow Airways R568.Change NAV1 to VOR MAU 117.90.Over VOR MAU turn to 267° for entry Airways A552. Start to descend to 3000 ft,20nm after VOR MAU and change again NAV1 to VOR MAR 115.70.With 2nm from VOR MAR turn downwind to 208° and continued descend to 1500 ft.With 6 nm from VOR MAR turn base with 275° and then after 2nm, 26° for get LOC, final and landing SVMC Rwy 02L(ALT 236 ft).ILS 110.10 available | 8000 |
| VE-041 "El Lechero" | Maracaibo La Chinita Intl Santa Barbara del Zulia | SVMC SVSZ | 117 57 | Set NAV1 to VOR MAR 115.70. Flight depart 07.00 hours.Dep Rwy 02L,26° for 4nm,then turn to 74° for 2nm and 109° for another 2 nm.After turn to 209°.With 11nm from VOR MAR turn to 193°HDG INT DALEX and follow Airways W1. Over INT Dalex and with 52.3 nm from VOR MAR turn to 198°.Change NAV1 to VOR STB 114.90. With 28nm from VOR STB start to descend to 2700 ft.With visual of the airfield,descend to 1500 ft and start lefhand pattern,turn to 176° for downwind,then base with 77° and final for the last 4nm with 358° in SVSZ Rwy 36(ALT 33 ft). | 8000 |
| VE-042 "El Lechero" | Santa Barbara del Zulia El Vigía Juan Pablo Perez Alfonso | SVSZ SVVG | 40 27 | Flight depart 08.20 hours.Dep.Rwy 36,358° for 3nm,then 58° for 1.5nm and 95° for another 1.5nm.After turn to 169° and set NAV1 to VOR STB 114.90 and ADF to NDB EVG 393.0.With 8.6nm from VOR STB start to descend to 1500 ft.With 23nm from VOR STB and with visual of the airfield turn to 138° for 2nm and descend to 1000 ft.Then turn 86° for final SVVG Rwy 09(ALT 249 ft) | 5000 |
| VE-043 "El Lechero" | El Vigía Juan Pablo Perez Alfonso La Fría | SVVG SVLF | 62 33 | Flight depart 09.10. Dep.Rwy 09,87° for 3nm,then 134° for 1.5nm and 185° for another 1.5nm.After turn to 249° and set NAV1 to VOR LFA 113.00.Start to descend to 2800 ft 12nm from VOR LFA.With 1nm in radial from VOR LFA and visual of the airfield start lefthand pattern turning downwind to 185° for 5.6nm and descending to 1500 ft.With 4.5nm from VOR LFA, turn base to 95° for 1.5nm and finally turn to 5° to the last 4nm for final SVLF Rwy 36(ALT 305 ft). | 6000 |
| VE-044 "El Lechero" | La Fría San Antonio del Táchira | SVLF SVSA | 37 27 | Flight depart 10.05. Dep.Rwy 36,2°,then 55° for 1.5nm, 95° for another 1.5nm and 185° for 5nm.After turn to 224° HDG INT Babor.Set NAV1 to VOR CUC 113.30. With 17nm from VOR CUC start slow descend to 2000 ft .Over INT Babor and with 2.5nm from VOR CUC radial,turn to 161° for final SVSA Rwy 16(ALT 1312 ft). | 6000 |
| VE-045 "El Lechero" | San Antonio del Táchira Santo Domingo-Mayor Buenaventura Vivas Intl | SVSA SVSO | 32 26 | Flight depart 10.55. Dep.Rwy 16,162° for 5nm,then turn to 131°.Set NAV1 to DME STD 115.30.With 17.6nm from DME STD start slow descend to 4000 ft.With 13nm turn to 126° and continued descend to 2000 ft.With 5 nm from DME STD and visual of the airfield turn to 110° for final in SVSO Rwy 11(ALT 1083 ft). | 5000 |
| VE-046 "El Occidental" | Santo Domingo-Mayor Buenaventura Vivas Intl San Antonio del Táchira | SVSO SVSA | 50 32 | Flight depart 13.30.Dep Rwy 11,109° for 3nm,then turn to 160° for 1.5nm,205° for again 1.5nm and 256° for anothers 1.5nm. After turn to 312° and set NAV1 to VOR CUC 113.30.With 15.3nm from VOR CUC start slow descend to 3400 ft.With 5.8 nm from VOR CUC turn to 336° for starting righthand pattern downwind and continued descend to 2400 ft.With 2.5nm from VOR CUC turn base with 72° for 2nm HDG INT Babor.With 2.1nm from VOR CUC and over INT Babor turn to 161° for final SVSA Rwy 16(ALT 1312 ft). | 6000 |
| VE-047 "El Occidental" | San Antonio del Táchira La Fría | SVSA SVLF | 42 28 | Flight depart 14.25.Set NAV1 to VOR CUC 113.30.Dep.Rwy 16,161° for 3nm,then turn to 252° for 1.5nm and then 347°.With 1.5nm from VOR CUC turn to 42°to follow Airways W34 and change NAV1 to VOR LFA 113.00.With 13nm from VOR LFA start slow descend to 2400 ft.With 2.3nm from VOR LFA turn to 5° for start righthand pattern downwind and descend to 1500ft. With 3.1 nm from VOR LFA turn to 95° for base and then with 3.3nm from VOR LFA turn 185° for final and landing SVLF Rwy 18(ALT 305 ft). | 5000 |
| VE-048 "El Occidental" | La Fría El Vigía Juan Pablo Perez Alfonso | SVLF SVVG | 54 33 | Flight depart 15.15.Set NAV1 to VOR LFA 113.00 and ADF to NDB EVG 393.0.Dep.Rwy 18,183° for 3nm,then turn to 275° for 2nm and 5° for 5nm.With 3.2nm from VOR LFA turn 39° HDG INT Tabla. Over INT Tabla and with 16 nm from VOR LFA turn to 78°.Start to descend to 1200 ft,22.7nm from VOR LFA .With 37.6nm from VOR LFA turn to 86° for final in SVVG Rwy 09(ALT 249 ft). | 5000 |
| VE-049 "El Occidental" | El Vigía Juan Pablo Perez Alfonso Santa Barbara del Zulia | SVVG SVSZ | 36 24 | Flight depart 16.10.Dep.Rwy 09,87° for 3.5nm, then turn to 43° for 1.5nm and after 319°.Set NAV1 to VOR STB 114.90.With 15nm from VOR STB start descend to 1500 ft.With 2nm from VOR STB continued descend and entry final for the last 4nm with 358° in SVSZ Rwy 36(ALT 33 ft). | 4000 |
| VE-050 "El Occidental" | Santa Barbara del Zulia Maracaibo La Chinita Intl | SVSZ SVMC | 104 53 | Flight depart 17.15.Plane catering.Set NAV1 to VOR STB 114.90.Dep.Rwy 36,358° for 4nm. With 6.4nm from VOR STB turn to 16° HDG INT Bobat and follow Airways W1.Change NAV1 to VOR MAR 115.70.Over INT Bobat and with 40.5 nm from VOR MAR turn to 10°.With 36nm from VOR MAR start descend to 1500 ft.With 7.5nm from VOR MAR turn to 26° to entry in final and landing SVMC Rwy 02L(ALT 236 ft).ILS 110.10 available | 9000 |
| VE-051 | Barcelona,Gen. Jose Antonio Anzoategui Carúpano,Gen.Jose Francisco Bermudez | SVBC SVCP | 111 53 | Dep.Rwy 20,198° for 4nm, then turn to 145° for 1.5nm and after 63°.Set NAV1 to VOR CUM 113.30.Over VOR CUM turn to 89° to follow Airways A552 and set ADF to NDB CUP 270.0.Start to descend to 2000 ft,36.5nm after VOR CUM.With 55.5 nm from VOR CUM and airfield in sight start lefthand pattern turning to 352° for 5nm downwind, then base with 261° for 1.5nm and finally turn to 169° for final and landing SVCP Rwy 17(ALT 33 ft). | 7000 |
| VE-052 | Carúpano,Gen.Jose Francisco Bermudez Barcelona,Gen. Jose Antonio Anzoategui | SVCP SVBC | 99 48 | Dep.Rwy 17,171° for 3nm,then 192° for 1.5nm and 240° for another 1.5nm.After turn to 293° and set NAV1 to VOR CUM 113.30.With 42.5nm from VOR CUM turn to 267° to follow Airways A552. Over VOR CUM turn to 251° and follow now Aiways W12. Start descend to 1800 ft, 8nm after VOR CUM.Change NAV1 to VOR BNA 115.90.With 6.6nm from VOR BNA turn to base with 280° and continued descend to 1000 ft and then turn to 198° for final SVBC Rwy 20(ALT 26 ft). | 8000 |
|  | SERVICIO INTERNACIONAL |  |  | INTERNATIONAL FLIGHTS |  |
| VE-100a "Caribbean Moon" | Maiquetia Simon Bolivar Intl San Juan Luis Muñoz Marin Intl | SVMI TJSJ | 488 194 | Set NAV1 to VOR MIQ 114.80 Night flight departing 22.00 hours.Dep Rwy 09, 96°, after 71° for 2nm and then 11° HDG INT DARPA. Over INT DARPA and with 17nm from VOR MIQ turn to 22° to follow Airways G432 and change NAV1 to VOR LRS 113.10.Over VOR LRS turn to 9° HDG INT SILVA.Over INT SILVA and with 151 nm from VOR LRS turn to 14° HDG INT ALASK and change NAV1 to VOR PSE 109.00.Over INT ALASK and with 69.5nm from VOR PSE small correction to 11°. Over VOR PSE turn to 55° to follow Airways RTE9.Start slow descend to 1000 ft, 4.4 nm after VOR PSE.Set ADF to NDB SJ 330.0.Over NDB SJ turn to 78° for final and landing TJSJ Rwy 08(ALT 10 ft).ILS 110.30 available. | 9000 |
| VE-100b "Caribbean Moon" | San Juan Luis Muñoz Marin Intl Providenciales Intl. | TJSJ MBPV | 428 178 | Set NAV1 to VOR SJU 114.00 Flight depart 03.30 hours.Dep. Rwy 10, 101° for 4nm,then 58° for 2 nm and 352° for again 2 nm.Then HDG 302° HDG INT CORAF and Airways RTE6.Over INT CORAF and with 26 nm from VOR SJU turn to 308°HDG INT IDAHO and follow Airways RTE6. Change NAV 1 to VOR BQN 113.50. Over INT IDAHO and with 54.5nm from VOR BQN turn to 313°HDG INT GRADI and follow now Airways A555.Set ADF to NDB GT 232.0 and NAV1 to VOR GTK 114.20.Over VOR GTK turn to 296°to follow now airways G648 and change NAV1 to VOR PVN 115.60.Start to descend to 5200 ft,18.4 nm from VOR PVN.With 9.6nm from VOR PVN turn to 313° HDG INT ZORON to start Lefthand traffic pattern downwind.Continued descend to 3900 ft .Over INT ZORON and 5.7 nm from VOR PVN turn to 8° and continued descend to 2200 ft.With 1.5nm from radial VOR PVN and in downwind turn to 282° for 4.5nm.With 5nm from VOR PVN turn base to 197°for 1.5nm and then 103° for the last 4nm to entry final and landing in MBPV Rwy 10(ALT 16 ft). | 8000 |
| VE-100c "Caribbean Moon" | Providenciales Intl. Miami Intl | MBPV KMIA | 530 211 | Set NAV1 to VOR PVN 115.60 Dep.Rwy 10,101° for 3nm,then 60° for 1.5nm and 335° HDG INT MICAS. Over INT MICAS and with 23.6nmfrom VOR PVN turn to 307°HDG INT INDEE and follow Airways A555.Set ADF to NDB ZLS 526.0 and NAV1 to VOR ZQA 112.70.Over INT INDEE and 55nm from NDB ZLS small HDG correction to 305°.Over NDB ZLS turn to 312° HDG INT GEROT,47nm from NDB ZLS and 100 nm from VOR ZQA.Over VOR ZQA turn to 297° HDG INT RAJAY.Over INT RAJAY with 43.4nm from VOR ZQA,turn to 296° and set NAV1 to VOR ZBV 116.70.Cheking VOR ZBV and turn to 261°and change NAV1 to VOR VKZ 117.10.With 28.8nm from VOR VKZ turn to 288° to follow Airways BR35V.Start descend to 4200 ft, 20nm from VOR VKZ. Over VOR VKZ turn to 275° and starting righthand pattern downwind with 13nm and continued descend to 1200 ft.With 12.9nm from VOR VKZ turn to 2° HDG INT DOXSI for 1.5nm.Then turn to 91° to capture LOC and entry final and landing KMIA Rwy 09R(ALT 7 ft).ILS 110.90 available | 10000 |
| VE-101a "Caribbean Sun" | Miami Intl Providenciales Intl. | KMIA MBPV | 515 209 | Set NAV1 to VOR VKZ 117.10 Flight depart 11.30.Dep.Rwy 12, 123° for 4nm,then HDG 104° to VOR VKZ.Over VOR VKZ turn to 98° and change NAV1 VOR ZBV 116.70.Checking VOR ZBV turn to 117° to entry Airways A555.Change NAV1 to VOR ZQA 112.70.Over VOR ZQA turn to 132° to follow Airways A555 and set ADF to NDB ZLS 526.0.Over NDB ZLS turn to 127° HDG INT MICAS and set NAV1 to VOR PVN 115.60.Start slow descend to 1500 ft 32nm from VOR PVN.Over INT MICAS and with 24nm from VOR PVN turn to 174° HDG INT FF10 and continued descend.Over INT FF10 and with 4.5nm from VOR PVN turn to 103°for final and landing MBPV Rwy 10(ALT 16 ft). | 9000 |
| VE-101b "Caribbean Sun" | Providenciales Intl. San Juan Luis Muñoz Marin Intl | MBPV TJSJ | 410 173 | Set NAV1 to VOR PVN 115.60 Dep.Rwy 10,101° with 3.2nm HDG to INT WESNA.Over INT WESNA and with 2.7nm from VOR PVN,turn to 120° and set ADF to NDB SC 260.0.Over NDB SC and with 43.6nm from VOR PVN ,turn to 109°and change NAV 1 to VOR GTK 114.20. Over VOR GTK turn to 132° HDG INT GRADI and follow Airways A555.Over INT GRADI and with 100 nm from VOR GTK small HDG correction to 134° HDG INT IDAHO.Change NAV1 to VOR BQN 113.50.Over INT IDAHO and with 55nm from VOR BQN another small HDG correction to 135°.Change NAV1 to VOR SJU 114.00 and set ADF to NDB DDP 391.0.Start descend to 5300 ft ,29 nm from VOR SJU.Over NDB DDP turn to 110°HDG INT BINJI and continued descend to 2200 ft. Checking INT BINJI with 11.5nm from VOR SJU and entry in final to capture LOC with 101° and landing TJSJ Rwy 10(ALT 10 ft).ILS 109.70 available | 7000 |
| VE-101c "Caribbean Sun" | San Juan Luis Muñoz Marin Intl Maiquetia Simon Bolivar Intl | TJSJ SVMI | 512 204 | Flight depart 18.20.Dep. Rwy 08, 78° for 4nm,then 114° for 2 nm and after 241° to follow Airways RTE9.Set NAV1 to VOR PSE 109.00. Over VOR PSE turn to 191°HDG INT ALASK and follow now Airways G432.Over INT ALASK and with 69.5nm from VOR PSE small HDG correction to 193°HDG INT SILVA.Change NAV1 to VOR LRS 113.10.Over INT SILVA and with 150nm from VOR LRS turn to 189°. Over VOR LRS turn to 202°HDG INT DARPA. Start descend to 8000 ft , 61 nm from VOR LRS.Then change NAV1 to VOR MIQ 114.80. Over INT DARPA and with 17nm from VOR MIQ turn to 232° and continued descend to 2800 ft.With 11 nm from VOR MIQ turn small base with 191° HDG INT MIQ11 and then to 96° for get the LOC ,final and landing SVMI Rwy 09(ALT 236 ft).ILS 110.10 available | 10000 |
| VE-102 | Maiquetia Simon Bolivar Intl Willemstad-Hato Intl | SVMI TNCC | 166 72 | Dep Rwy 08, 82° for 3nm, then turn to 36° for 2nm and after 315°.Set VOR to PJG 116.70 and start descend to 1300 ft ,26nm from 116.70. With 2.5 nm from 116.70 turn for base with 18°(2nm) and then 110° for final and landing TNCC Rwy 11(ALT 30 ft).ILS 110.90 available | 8000 |
| VE-103 | Willemstad-Hato Intl Maiquetia Simon Bolivar Intl | TNCC SVMI | 162 66 | Dep.Rwy 11, 110° for 5 nm,then 164°.Set NAV1 to VOR PNA 112.90.After VOR PNA turn to 129° HDG INT TORIM and follow Airways G466. Change NAV1 to VOR MIQ 114.80.Over INT TORIM and with 34.7nm from VOR MIQ turn to 113° HDG INT MIQ14.With 28.5nm from VOR MIQ start descend to 3000 ft.Over INT MIQ turn to 96° for get the LOC, final and landing SVMI Rwy 09(ALT 236 ft).ILS 110.10 available | 7000 |
| VE-104 | Willemstad-Hato Intl Orangestad Reina Beatrix Intl | TNCC TNCA | 76 35 | Flight depart 11.20.Dep.Rwy 29, 290° for 6nm,then 296° to follow Airways A574.Set NAV1 to VOR ABA 112.50.Start descend to 1600 ft, 10.2 nm from VOR ABA.Over VOR ABA turn to 296° for 8 nm and then 260°(1.5nm)and 192°(1.5nm) for base.Last turn to 112° for final and landing TNCA Rwy 11(ALT 59 ft) ILS 108.70 available. | 6000 |
| VE-105 | Orangestad Reina Beatrix Intl Willemstad-Hato Intl | TNCA TNCC | 77 36 | Set NAV1 to VOR ABA 112.50 Flight depart 14.10.Dep.Rwy 11, 111° for 5 nm,then 104°.With 6.3nm from VOR ABA turn to 117° and follow Airways A574. Then change NAV1 to VOR PJG 116.70.Start descend to 3200 ft 2.2nm from VOR PJG.Over VOR PJG turn to 65° for 5nm and continue descend to 1900 ft,then 111° for 5.5 nm.Base with 161°(2 nm) and 231° (2 nm).Last ,turn to 290° for final and landing TNCC Rwy 29(ALT 30 ft) ILS 111.90 available | 5000 |
| VE-106 | Maiquetia Simon Bolivar Intl Bogota El Dorado Intl | SVMI SKBO | 614 228 | Flight depart 08.15.Dep.Rwy 09, 96° for 4 nm,then 49°(2nm) and 347°(2nm).After HDG 269°.Set VOR to PBL 117.70.After VOR 117.70 turn to 258° and change VOR to BRM 112.20.After VOR 112.20 turn to 248° with HDG to VOR 114.40 VRA.Over VOR 114.40 turn to 239° and set ADF to NDB EVG 393.0. After NDB 393.0 turn to 243° and reset VOR to LFA 113.00.Then turn to 222° to entry in Colombian Air Space and set VOR to CUC 113.30. Over VOR 113.30 turn to 224° HDG VOR BGA 113.50.After VOR 113.50 ,turn to 206° and change VOR to BUV 116.20.Checking VOR 116.20 and turning to 217° ,HDG VOR BOG 113.90.Over VOR 113.90 turn to 132° and start descend to 9900 1 nm after the checking point.ADF set to NDB ED 244.0,then 130° for NDB DR 264.0 for landing SKBO Rwy 13L(ALT 8360 ft).ILS 109.90 available | 12000 |
| VE-107 | Bogota El Dorado Intl Maiquetia Simon Bolivar Intl | SKBO SVMI | 570 208 | Mix adjusted to SKBO ALT(8360 ft)* Flight depart 16.20.Dep.Rwy 13L, 130° with ADF set to NDB R 274.0. After NDB 274.0 turn to 93°(2 nm)and then 51°.ADF reset to NDB SOG 355.0.After NDB 355.0 turn to 34° and set ADF to NDB EPL 237.0.After NDB 237.0 turn to 64° and set VOR to BNS 117.50.Over 117.50 turn to 54° and ADF set to NDB GRE 318.0. After NDB 318.0 turn to 52° and set VOR to AGV 113.40.After VOR 113.40 turn to 73° and change VOR to VOR VAL 117.40.Start slow descend to 1740 ft,27 nm after VOR 117.40.Change VOR to MIQ 114.80 after BOD point .With 8.5 nm from VOR 114.80 turn to 94° for final and landing SVMI Rwy 09(ALT 236 ft) ILS 110.10 available. *Recommend very long Take off run. Wait in Rwy to build speed and climb very slowly(300-400 fpm).Mix recomended:Props 65 %,Mix 35 % | 13000 |
| VE-108 | Maiquetia Simon Bolivar Intl Las Americas Dr J.F.Peña Gomez *Flight operated by DC-4 | SVMI MDSD | 515 152 | Dep.Rwy 09,96° for 4nm,then 56° for 2nm and 10° for another 2 nm.After turn to 351° HDG INT ILKIT. Set NAV1 to VOR MIQ 114.80.Over INT ILKIT and 94nm from VOR MIQ turn to 349° and change NAV1 to VOR SIS 114.50.Start slow descend to 1500 ft 53nm from VOR SIS.Over VOR SIS and in Right hand pattern downwind, turn base to 81° for 3.6nm and then 169° for final and landing MDSD Rwy 17(ALT 59 ft). ILS 108.90 available | 10000 |
| VE-109 | Las Americas Dr J.F.Peña Gomez Nassau Intl. *Flight operated by DC-4 | MDSD MYNN | 598 173 | Dep.Rwy 17,169° for 4nm,then turn to 229° for 1.5nm and 278° for again 1.5nm.After turn to 344° and set NAV1 to VOR SIS 114.50.Over VOR SIS turn to 327° and change NAV1 to VOR SGO 114.90.Over VOR SGO turn to 321° and set ADF to NDB ZLS 526.0.Checking NDB ZLS and turn to 312° to follow Airways A555 .Set NAV1 to VOR ZQA 112.70.Start to descend to 1500 ft, 50.5nm from VOR ZQA.Over INT DILPE and 4.6nm from VOR ZQA turn to 320° for final and landing MYNN Rwy 32(ALT 84 ft).ILS 110.10 available. | 8000 |
| VE-110 | Nassau Intl. Atlanta Hartfield Intl *Flight operated by DC-4 | MYNN KATL | 646 194 | Set NAV1 to VOR ZQA 112.70 Dep.Rwy 32,320° for 5nm,then turn to 335° HDG INT ZQA33.Over INT ZQA33 and with 14nm from VOR ZQA turn to 328° to follow Airways BR65V.Change NAV1 to VOR ZFP 113.20.Over VOR ZFP turn to 330° and change NAV1 to VOR SGJ 109.40.Over VOR SGJ turn to 324° and change NAV1 to VOR AYS 110.20.Over VOR AYS turn to 327° and set ADF to NDB MC 290.0.Over NDB MC turn to 324° and change ADF to NDB JHH 412.0 and NAV1 to VOR ATL 116.90.Start to descend to 2200 ft ,41nm from VOR ATL.Checking NDB JHH and continued in HDG 224°.With 8.5nm from radial VOR ATL turn to 2° for 2nm and then 92° for entry in final,capture LOC and landing KATL Rwy 09L(ALT 1027). ILS 110.50 available. | 10000 |
| VE-111 | Atlanta Hartfield Intl Nassau Intl. *Flight operated by DC-4 | KATL MYNN | 727 212 | Dep.Rwy 08L,92° for 7nm,then turn to 140° and set ADF to NDB DB 251.0.Over NDB DB turn to 149° and change ADF to NDB BHC 376.0.Over NDB BHC turn to 151° and set NAV1 to DME NRB 111.40.Check DME NRB, turn to 152° and change NAV 1 to VOR ZFP 113.20.Over VOR ZFP mantain HDG 152° and set ADF to NDB ZCC 302.0.Start to descend to 7400 ft, 67 nm after VOR ZFP.Over NDB ZCC turn to 144° ,continued descend to 1500 ft and change NAV1 to VOR ZQA 112.70.With 7nm from radial VOR ZQA and in lefthand pattern downwind, turn base with 57° for 3nm HDG INT FS32.Over FS32,turn to 321° for final and landing MYNN Rwy 32(ALT 16 ft).ILS 110.10 available | 9000 |
| VE-112 | Nassau Intl. Las Americas Dr J.F.Peña Gomez *Flight operated by DC-4 | MYNN MDSD | 626 184 | Dep.Rwy 32,320° for 5nm,then turn to 275° for 2nm and 225° for another 2nm.After turn HDG 139° and set ADF to NDB ZEF 328.0.Over NDB ZEF turn to 148° and change ADF to NDB ZIN 376.0.Cheking NDB ZIN,turn to 139° and set NAV1 to VOR HCN 113.90.Over VOR HCN turn to 126° and change NAV1 to VOR SIS 114.50.Start slow descend to 3000 ft,46 nm from VOR SIS.Over VOR SIS turn to 171° for start a lefthand pattern downwind. Continued descending to 1200 ft. With 12nm from VOR SIS turn base with 82° for 4nm.Then turn 349° for final and landing MDSD Rwy 35(ALT 59 ft).ILS 108.90 available | 11000 |
| VE-113 | Las Americas Dr J.F.Peña Gomez Maiquetia Simon Bolivar Intl *Flight operated by DC-4 | MDSD SVMI | 553 168 | Set Nav1 to VOR SIS 114.50. Dep.Rwy 35, 349° for 6nm,then turn to 299° for 2nm and 249° for again 2nm.Over VOR SIS turn to 181° and after change NAV1 to VOR PJG 116.70.Over VOR PJG turn to 159° and change NAV1 to VOR PNA 112.90. Over VOR PNA turn to 129° HDG INT TORIM and follow Airways G446.Start slow descend to 5100 ft ,36.5nm from VOR PNA.Over INT TORIM.Change NAV1 to VOR MIQ 114.80.Over INT TORIM and with 34.7nm from VOR MIQ turn to 113° HDG INT MIQ14 and continued descend to 2000 ft.Over MIQ14 and with 14nm from VOR MIQ turn to 96 for capture LOC, final and landing SVMI Rwy 09(ALT 236 ft). ILS 110.10 available | 9000 |
| VE-114 | Maiquetia Simon Bolivar Intl Norman Manley Intl-Kingston *Flight operated by DC-4 | SVMI MKJP | 727 209 | Dep.Rwy 09,96° for 4nm, then turn to 55° for 1.5nm and 9° also for 1.5nm.Then turn to 315° and set NAV1 to VOR PJG 116.70.Over VOR PJG turn to 312° and turn NAV1 to VOR MLY 115.50.Start descend to 1500 ft, 62nm from VOR MLY.With 6nm from VOR MLY and over INT AVREL turn to 296° for final and landing MKJP Rwy 30(ALT 10 ft). | 10000 |
| VE-115 | Norman Manley Intl-Kingston La Havana Jose Marti Intl. *Flight operated by DC-4 | MKJP MUHA | 455 132 | Dep.Rwy 30,296° for 4nm,then turn to 301° and set NAV1 to VOR SIA 115.70.Over VOR SIA turn to 306° and set ADF to NDB CBC 415.0.Cheking NDB CBC and turn to 322° and change NAV1 to VOR UCL 112.10.Over VOR UCL turn to 326° and set NAV1 to VOR UHA 116.10.Start to descend to 1500 ft,44 nm from VOR UHA.With 6nm from VOR UHA and airfield in sight, turn to 56°for entry final, capture LOC and landing MUHA Rwy 06(ALT 210 ft). ILS 110.50 available | 8000 |
| VE-116 | La Havana Jose Marti Intl. Norman Manley Intl-Kingston *Flight operated by DC-4 | MUHA MKJP | 475 142 | Dep.Rwy 06,56° for 5nm then turn to 109° and set ADF to NDB UZG 283.0.Over NDB UZG turn to 162° and set NAV1 to VOR UCL 112.10.Over VOR UCL turn to 143° and change ADF to NDB CBC 415.0.Checking NDB CBC and turn to 126°.Set NAV1 to VOR SIA 115.70.Over VOR SIA turn to 123° and start descend to 1500 ft,23 nm after checking.Then change NAV1 to VOR MLY 115.50.With 7nm from VOR MLY and in Lefthand pattern downwind turn base with 31° HDG INT AVREL for 2.5nm.Over INT AVREL turn to 296° for final and landing MKJP Rwy 30(ALT 10 ft). | 11000 |
| VE-117 | Norman Manley Intl-Kingston Maiquetia Simon Bolivar Intl *Flight operated by DC-4 | MKJP SVMI | 737 216 | Flight departing time:18.10.Dep.Rwy 30,296° for 5nm,then turn to 253° for 2nm and 183° for another 2nm.After HDG 135°.Set NAV1 to VOR ABA 112.50.Climb to 11.500 ft after dinner and with passengers asleep;check passenger comfort with Flight Attendants. Ev.descend to 10.000 ft.Over VOR ABA turn to 140° and change NAV1 to VOR PNA 112.90.Over VOR PNA turn to 126° HDG INT ROMEX.Start slow descend to 3500 ft ,22.5nm after VOR PNA.Over INT ROMEX and with 73 nm from VOR PNA turn to 121° HDG INT MIQ14 and descend to 2200 ft.Change NAV1 to VOR MIQ 114.80.Over MIQ14 and with 14nm from VOR MIQ turn to 96 for capture LOC, final and landing SVMI Rwy 09(ALT 236 ft). ILS 110.10 available | 11500 |
| VE-118 | Maiquetia Simon Bolivar Intl Guadeloupe-Pointe-A-Pitre | SVMI TFFR | 497 199 | Dep.Rwy 09,96° for 4nm,then turn 57°to FD27 INT for 3nm and after turn to 61°.Set ADF to NDB DCF 260.0 and NAV1 to VOR PPR 112.90.Over NDB DCF turn to 29° and change ADF to NDB MG 376.0.Start to descend to 2000 ft,25.4 nm from VOR PPR.Over NDB MG turn to 360°.With 12 nm from radial VOR PPR turn to 294° for final and landing TFFR Rwy 29(ALT 36 ft). ILS 110.30 available | 7000 |
| VE-119 | Guadeloupe-Pointe-A-Pitre Maiquetia Simon Bolivar Intl | TFFR SVMI | 499 200 | Flight departing time:16.20. Dep.Rwy 29,294° for 5nm,then turn to 239°.Set NAV1 to VOR LRS 113.10.Over VOR LRS turn to 214° HDG INT MARES.Start slow descend to 6000 ft ,63,5nm from VOR LRS.Over INT MARES and with 76.5nm from VOR LRS turn to 193° HDG INT MIQ14 and change NAV1 to VOR MIQ 114.80.Over MIQ14 and with 14nm from VOR MIQ 114.80 turn to 96° for final and landing SVMI Rwy 09(ALT 236 ft). ILS 110.10 available | 10000 |
| VE-120 | Maiquetia Simon Bolivar Intl Piarco Intl-Port Of Spain | SVMI TTPP | 353 142 | Dep.Rwy 09,96° for 5nm,then turn to 95°.Set NAV1 to VOR CBC 113.50.With 26nm from VOR CBC turn to 109°.Over VOR CBC turn to 105° to follow Airways A552 and change NAV1 to VOR CUM 113.30.Over VOR CUM turn to 88° and set ADF to NDB CUP 270.0.Cheking NDB CUP and turn to 109°always in the Airways A552.Set NAV1 to VOR POS 116.90. Start descend to 4300 ft,16.3nm from VOR POS.Over VOR POS turn to 79° for 10nm and continued descend to 2000 ft.Then turn to 13° for 3.7nm. With 12nm from VOR POS turn to 283° for final and landing TTPP Rwy 28(ALT 59 ft).ILS 109.70 available. | 9000 |
| VE-121 | Piarco Intl-Port Of Spain Maiquetia Simon Bolivar Intl | TTPP SVMI | 356 142 | Set ADF to NDB POS 382.0.Dep.Rwy 28,283° HDG NDB POS for 5.9nm.Then turn to 284° and change ADF to NDB CUP 270.0.Over NDB CUP turn to 267° to follow Airways A552 and set NAV1 to VOR CUM 113.30.Checking VOR CUM and turn to 284°always in the Airways A552,change NAV1 to VOR CBC 113.50.Over VOR CBC turn to 296° HDG INT OSMAR.Over INT OSMAR and with 31.6nm from VOR CBC turn to 276° for a leftthand pattern.Start descend to 3050 ft 37.6nm after VOR CBC.Change NAV1 to VOR MIQ 114.80.After 26nm and 8nm from radial VOR MIQ,turn to 225° for 2.7 nm and base to 188° with 4nm.With 7nm from radial VOR MIQ, turn to 96° for final and landing SVMI Rwy 09(ALT 236 ft).ILS 110.10 available | 10000 |
| VE-122 | Maiquetia Simon Bolivar Intl Grenada-Point Salines Intl. | SVMI TGPY | 334 134 | Dep.Rwy 09,96° for 6nm,then turn to 80° for a OSMAR 09 DEP.Over INT OSMAR turn to 96° and set NAV1 to VOR MTA 114.10.Over VOR MTA turn to 75° HDG INT DAREK and follow Airways A561.Over INT DAREK and with 76.3nm from VOR MTA turn to 79° and change NAV1 to VOR GND 117.10.Start to descend to 1500 ft,25.5nm from VOR GND.With 5nm from VOR GND and airfield in sight, turn to base with 12° for 2.5nm and then 282° for final and landing TGPY Rwy 28(ALT 39 fr). | 9000 |
| VE-123 | Grenada-Point Salines Intl. Maiquetia Simon Bolivar Intl | TGPY SVMI | 340 136 | Flight departing time:17.15.Dep.Rwy 28,283° for 5nm,then turn to 253° HDG INT DAREK.Set NAV1 to VOR MTA 114.10.Over INT DAREK and with 76.3nm from VOR MTA turn to 255°.Over VOR MTA turn to 270° HDG INT DAGOX.Checking INT DAGOX with 96.5nm from VOR MTA and turn to 284° HDG INT OSMAR.Change NAV1 to VOR MIQ 114.80.With 26nm from VOR MIQ and over INT OSMAR turn to 269°.Start to descend to 2000 ft, 16nm from VOR MIQ.With 8nm from VOR MIQ turn to base with 190° for 2nm and then 96° for final and landing SVMI RWy 09(ALT 236 ft). ILS 110.10 available | 8000 |