*********DC-3 POWER SETTINGS*********

_________________________________________________________________________

Throttle sequence is PROPS, POWER, MIXTURE

TAKE OFF:           2700 rpms - 48 inches MAP - AUTO RICH (Trim -2 degrees)

ROTATE                           84 knts

CLIMB OUT                   400 fpm - 105 knts

LANDING GEAR up at   500AGL

POWER SETTINGS:

Take-Off               2700 rpms - 48 inches MAP - AUTO RICH (Cowl Flaps out) & (Tail Wheel Locked)

95 knts                  2550 rpms - 40 inches MAP - AUTO RICH (Trim Down) 110 knts                2350 rpms - 36 inches MAP - AUTO RICH (Cowl Flaps trail)

Cruise 140/150 knts    2050 rpms - 30 inches MAP - AUTO LEAN (Cowl Flaps Trail)

STANDARD RATE OF TURN: 30 Degree Bank Angle (Keep Ball [turn & slip] in the Middle)

_________________________________________________________________________

DECENT at           400fpm ..Reduce Power only. (Cowl Flaps Trail)

Throttle sequence is POWER, PROPS, MIXTURE

PATTERN SPEED:      110 knts (Cowl Flaps Trail)

GEAR DOWN             140 knts (Trim Up)

20   degrees Flaps         135 knts (Trim Up)

30   degrees Flaps          99 knts

40   degrees Flaps          97 knts

FULL degrees Flaps      95 knts

APPROACH           2050 rpms - 24 inches MAP - AUTO LEAN  105 knts (Cowl Flaps out)

2050 rpms - 18 inches MAP - AUTO LEAN    90 knts (Cowl Flaps out)

OUTER MARKER  2700 rpms,  & ..Reduce Power, AUTO RICH (Cowl Flaps out)

LAND:                   84 knts    Over the Fence - Close Throttle   (Cowl Flaps out) & (Tail Wheel Locked)

74 knts ...MAIN WHEELS (Cowl Flaps in)

STALL SPEEDS:     FLAPS UP           67 knts

FLAPS DOWN   61 knts